BORDER Program User Guide

Contents:

System requirements							Page 2

Source data requirements							Page 3

System requirements

To run the BORDER program on your PC, you need:

-	An x86 computer, minimum 486DX66, Pentium recommended

-	Windows 95 or Windows NT  operating system

-	At least 8 MB memory (Windows 95), 16 MB (Windows NT), 32 MB or

more recommended

-	An SVGA monitor set up to (at least) 800 * 600 pixels and 16 colours

-	A mouse or compatible pointing device

-	At least 10 MB free disk space (depending on the file size of the line data)

-	Normal fonts installed (MS Sans Serif and Courier New are used)

Source data requirements

To use the BORDER program you need:

-	The border line data of your own country

-	The data of the borderlines of neighbouring countries in a distance of app. 80 km

All data have to be pairs of geographically co-ordinates (WGS84) in decimal form, e.g. 8 degrees and 30 minutes = 8.50. The data have to be stored in text format. One line of text contains one pair of co-ordinates, longitude first, latitude second. Longitude and latitude have to be separated by either one or more blanks, a comma or a semicolon. One line of text is limited to a length of max. 80 characters.

Example:

14.3332		52.2345

14.3102		52.2013

14.2989		52.1833

.

.

.

For border line data, first you have to split up your own border line data into parts belonging to your neighbouring countries. Please store this data using different names. Then you have to select parts (80 km) of the border lines coming from your neighbours (borders between neighbouring countries) reaching to your own border line. Please store those parts with own file names.

If you have one or more exclave(s) (exclave =  a portion of your country separated from the main part of your country), please store those data separately.

For border lines used in the HCM program you have to generate source files consisting of:

A part (80 km) of borders between your neighbouring countries at the beginning of your own border line,

your own border line,

a part (80 km) of borders between your neighbouring countries at the ending of your own border line and, in case you have exclaves,

the line data of the exclaves.

You have to ensure, that the resulting line contains only data starting at one point and going continuously to an other point. To solve this problem in cases, where your data is combined from different sources, you can use the menu item „Create an up side down copy of line data“ in the BORDER program to get the continuously order of the resulting file.

For border lines, the BORDER program needs the resulting line (combined of the three parts described above) and the data of exclaves. You can combine up to 9 source files in the menu item „Store line data in the HCM format“. Before you do it, please use menu item number 1, „Examine line data“ to check each input file. Use the default setting for filling gaps (500m). Only in cases the border line is over sea, the gap between two points can be greater.

To generate x-km lines, you need a different input file. This file normally consists only of  your own border line data (see the new Annex 13 of the HCM documentation!). If you have exclaves, the data of those exclaves have to be included in this input file. To do it, you have to cut the own border line in two parts at the nearest point to the exclave. Then you have also to cut the data of the exclave at the nearest point to the border line and to combine those three parts in continuous order to the new file used for the x-km calculation.

For lines used for the cross border range calculation, you need data of your own border line only and the data of the exclaves (like the border lines, but without parts between neighbouring countries).