Annex 6.2.

For the following  main subjects

- radio-frequency channel arrangements for FS systems including bands for PtP and PmP (FWA, BWA, WAS) systems

- coexistence and sharing conditions for FS towards different applications and radio services

- equipment characteristics for FS

- further aspects for FS reference is done to the relevant existing documentation of international organisations systematically e.g by taking frequency ranges into account.

The deliverables cover the following types of documents:

- ITU-R deliverables

ITU-R Recommendations F. Series

CEPT/ ECC/ERC deliverables

Decisions (DEC); Recommendations (REC); Technical Recommendations (T/R); Reports (REP)

ETSI deliverables 	European Norms (EN); Technical Standards (TS)

Table 1.: Reference list for channel Arrangements for FS including  CEPT/ECC/ERC  deliverables and corresponding ITU-R Recommendations  sortied by frequency bands

| Band designation (GHz) | Frequency band (MHz) | CEPT/ECC/ERC deliverable | Date of last Revision | ITU-R Recommendation | Date of last Revision |
| --- | --- | --- | --- | --- | --- |
| 1.4 | 1350-1375 and 1492-1517 | T/R 13-01 (Annex A) (Channel arrangements) | 2010 | F.1242-0 (Channel arrangements) | 1997 |
| 1.4 | 1375-1400 and 1427-1452 | T/R 13-01 (Annex B) (Channel arrangements) | 2010 | F.1242-0 (Channel arrangements) | 1997 |
| 2 | 2025-2110 and 2200-2290 | T/R 13-01 (Annex C) (Channel arrangements) | 2010 | F.1098-1 (Channel arrangements) | 1995 |
| 2.5 | 2520-2670 | (Note 1) | - | F.1243 (Channel arrangements) | 1997 |
| 3.5 | 3400-3600 | ERC/REC 14-03 (Channel arrangements and block assignment) | 1997 | F.1488-0 (Annex 2, only 0.25 MHz basic slots) (Block assignment) | 2000 |
| 3.7 | 3600-3800 | ERC/REC 12-08 (Annex B - part 2) (Channel arrangements and block assignment) | 1998 | F.1488-0 (Annex 2, only 0.25 MHz basic slots) (Block assignment) | 2000 |
| 4 | 3800-4200 | ERC/REC 12-08 (Annex B - part 1) (Channel arrangements) | 1998 | F.382-8 (Channel arrangements) | 2006 |
| 4 | 4400-5000 | - | - | F.1099 (Channel arrangements) | 2007 |
| 5 | 5725-5875 | ECC/REC/(06)04 | 2006 | F. 1399 |  |
| 6L | 5925-6425 | ERC/REC 14-01 (Channel arrangements) | 2007 | F.383-8 (Channel arrangements) | 2007 |
| 6U | 6425-7125 | ERC/REC 14-02 (Channel arrangements) | 2009 | F.384-11 (Channel arrangements) | 2012 |
| 7 | 7125-7425 | ECC/REC/(02)06 (Annex 1.1 or 2.1) (Channel arrangements) | 2011 | F.385-10 (Annex 1) (Channel arrangements) | 2012 |
| 7 | 7425-7725 | ECC/REC/(02)06 (Annex 1.1) (Channel arrangements) | 2011 | F.385-10 (Annex 1) (Channel arrangements) | 2012 |
| 7 | 7425-7900 | ECC/REC/(02)06 (Annex 2.2) (Channel arrangements) | 2011 | F.385-10 (Annex 4) (Channel arrangements) | 2012 |
| 8 | 7725-8275 | ECC/REC/(02)06 (Annex 1.2) (Channel arrangements) | 2011 | F.386-8 (Annex 6) (Not fully updated to ECC plan)(Note 2) (Channel arrangements) | 2007 |
| 8 | 7900-8500 | ECC/REC/(02)06 (Annex 2.3) | 2011 | F.386-8 (Does not contain ECC plan) | 2007 |
| 8 | 8275-8500 | ECC/REC/(02)06 (Annex 1.3) (Channel arrangements) | 2011 | F.386-8 (Annex 2) (Not fully updated to ECC plan)(Note 2) (Channel arrangements) | 2007 |
| 10 | 10000-10680 | ERC/REC 12-05 (Channel arrangements) | 2007 | F.747-1 Annex 3 (Channel arrangements) | 2012 |
| 11 | 10700-11700 | ERC/REC 12-06 (Channel arrangements) | 2010 | F.387-12 (Channel arrangements) | 2012 |
| 13 | 12750-13250 | ERC/REC 12-02 (Channel arrangements) | 2007 | F.497-7 (Channel arrangements) | 2007 |
| 14 | 14250-14500 | - | - | - | - |
| 15 | 14500-14620 and 15230-15350 | ERC/REC 12-07 (Channel arrangements) | 1996 | F.636-4 | 2012 |
| 18 | 17700-19700 | ERC/REC 12-03 (Channel arrangements) | 1994 | F.595-10 (Channel arrangements) | 2012 |
| 21 | 21200-21400 | - | - | F.637-4 (Annex 4) (Channel arrangements) | 2012 |
| 23 | 22000-22600 and 23000-23600 | T/R 13-02 (Annex A.1) (Channel arrangements) | 2010 | F.637-4 (Annex 2) (Channel arrangements) | 2012 |
| 23 | 22600-23000 | T/R 13-02 (Annexes A.2, A.3) (Channel arrangements) | 2010 | F.637-4 (Annex 2) (Channel arrangements) | 2012 |
| 26 | 24500-26500 | T/R 13-02 (Annex B) (Channel arrangements) ECC/REC/(11)01 (Note 3) (Block assignment) | 2010 | F.748-4 (Annex 1) (Channel arrangements) | 2001 |
| 28 | 27500-29500 | T/R 13-02 (Annex C) (Channel arrangements) ECC/REC/(11)01 (Note 3) (Block assignment) | 2010 2011 | F.748-4 (Annex 2) (Channel arrangements) | 2001 |
| 31 | 31000-31300 | ECC/REC/(02)02 (Channel arrangements) | 2010 | F.746-9 (Annex 7) (Channel arrangements) | 2009 |
| 32 | 31800-33400 | ERC/REC/(01)02 (Channel arrangements) ECC/REC/(11)01 (Note 3) (Block assignment) | 2010 2011 | F.1520-3 (Annex 1) (Channel arrangements) | 2011 |
| 38 | 37000-39500 | Recommendation T/R 12-01 (Channel arrangements) | 2010 | F.749-3 (Channel arrangements) | 2012 |
| 42 | 40500-43500 | ECC/REC/(01)04 (Channel arrangements and/or Block assignment) | 2010 | F.2005 (Channel arrangements and/or Block assignment) | 2012 |
| 50 | 48500-50200 | ERC/REC 12-10 (Channel arrangements) | 1998 | - | - |
| 52 | 51400-52600 | ERC/REC 12-11 (Channel arrangements) | 2001 | F.1496-1 (Channel arrangements) | 2002 |
| 55 | 55780-57000 | ERC/REC 12-12 (Channel arrangements) | 2001 | F.1497-1 (Channel arrangements) | 2002 |
| 60 | 57000-64000 | ECC/REC/(09)01 (Use of frequency band) | 2009 | - | - |
| 65 | 64000-66000 | ECC/REC/(05)02 (Use of frequency band) | 2009 | - | - |
| 70 and 80 | 71000-76000 and 81000-86000 | ECC/REC/(05)07 (Channel arrangements) | 2009 | F.2006 (Annex 2) (Channel arrangements) | 2012 |
| 90 | 92000-94000 and 94100-95000 | - | - | F.2004 (Channel arrangements) | 2012 |

Table 2.: Reference list for coexistance and sharing conditions for FS including  CEPT/ECC/ERC deliverables sortied by frequency bands

| Frequency band | Number | Title | Date |
| --- | --- | --- | --- |
| 3400-3600/3600-3800 MHz | ECC/DEC/(11)06 | Harmonised frequency arrangements for mobile/fixed communications network (MFCN) operating in the bands 3400-3600 MHz and 3600-3800 MHz | 2011 |
| 10.6-10.68 GHz | ECC/DEC/(10)01 | Sharing Conditions in the 10.6-10.68 GHz Band between FS, MS and EESS | 2010 |
| 3400-3800 MHz | ECC/DEC/(07)02 | Availability of frequency bands between 3400-3800 MHz for the harmonised implementation of BWA | 2007 |
| 1980-2010/2170-2200 MHz | ECC/DEC/(06)10 | Transitional arrangement for FS and TRR in 1980-2010/2170-2200 MHz | 2006 |
|  | ECC/DEC/(05)08 | High density applications in the Fixed-Satellite Service | 2005 |
| 27.5-29.5 GHz | ECC/DEC/(05)01 | 27.5-29.5 GHz by the FS and uncoordinated Earth stations of the FSS | 2005 |
|  | ECC/DEC/(04)10 | Designated for the temporary introduction of Automotive SRR | 2007 |
| 40.5-42.5 GHz | ECC/DEC/(02)04 | Terrestrial (fixed service/broadcasting service) systems and uncoordinated Earth stations in the fixed satellite service and broadcasting-satellite service (space to Earth) in the band 40.5-42.5 GHz | 2002 |
| 10.7-12.5 GHz | ERC/DEC/(00)08 | Use of 10.7-12.5 GHz by the Fixed and Broadcasting-satellite/Fixed-satellite Service | 2000 |
| 17.7-19.7 GHz | ERC/DEC/(00)07 | Shared use of 17.7-19.7 GHz for the Fixed and Fixed Satellite Service | 2000 |
| 37.5-40.5 GHz | ERC/DEC/(00)02 | 37.5-40.5 GHz for Fixed and Fixed Satellite Service | 2000 |
| 40.5-43.5 GHz | ERC/DEC/(99)15 | Harmonised frequency band 40.5-43.5 GHz for MWS including MWS and (P-P) FWS | 1999 |
| 7125-8500 MHz | ECC Report 163 | The usage of the 7125-8500 MHz band within the CEPT for the elaboration of the revision of the ECC/REC/(02)06 from version 2002 to version 2011 | 2011 |
| 5850-7075 MHz | ECC Report 156 | HAPS and other services/systems in 5850-7075 MHz | 2011 |
|  | ECC Report 127 | The impact of receiver standards on spectrum management | 2008 |
|  | ECC Report 126 | Fixed-Mobile Convergence | 2008 |
|  | ECC Report 125 | Guidelines for impact assessment | 2008 |
| 71-76 / 81-86 GHz | ECC Report 124 | Coexistence between Fixed Service operating in 71-76 / 81-86 GHz and the passive services | 2008 |
| 57-66 / 63-64 GHz | ECC Report 114 | Compatibility studies between MGWS in frequency range 57-66 GHz and other services/systems (except ITS in 63-64 GHz) | 2009 |
| 63 GHz | ECC Report 113 | Compatibility studies around 63 GHz between ITS and other systems | 2009 |
| 5725-5925 MHz | ECC Report 109 | Aggregate impact from ITS, BBDR and BFWA in the 5725-5925 MHz band on the other services/systems | 2007 |
| 3400- 3800 MHz | ECC Report 100 | Compatibility between BWA in the band 3400- 3800 MHz and other services | 2007 |
| 6 GHz | ECC Report 091 | Compatibility of ESV in the lower 6 GHz band | 2006 |
| 6 GHz | ECC Report 076 | Cross-Border coordination of Multipoint FWS in 3.4 to 33.4 GHz | 2006 |
|  | ECC Report 069 | Operation of Earth stations aboard vessels within the separation distances identified in ITU RR Resolution 902 | 2004 |
| 79 GHz | ECC Report 056 | Automotive collision warning SRR operating at 79 GHz | 2004 |
| 58 GHz | ECC Report 054 | EIRP of Terrestrial Fixed Links at around 58 GHz | 2004 |
| 23/26 GHz | ECC Report 046 | Immunity of 24 GHz automotive SRR from FS in 23/26 GHz | 2004 |
| 3.4-3.8 GHz | ECC Report 033 | Coexistence of FWA cells at 3.4-3.8 GHz | 2003 |
|  | ECC Report 032 | Improving co-existence of Multipoint FS | 2003 |
| 24 GHz | ECC Report 023 | Compatibility of 24 GHz Automotive Radars with FS, EESS, Radio Astronomy | 2003 |
| 24 GHz | ECC Report 020 | Methodology to determine the density of Fixed Service | 2002 |
|  | ECC Report 019 | Fixed Service for UMTS/IMT-2000 infrastructure | 2002 |
| 10.6-10.7 GHz | ECC Report 018 | Sharing - RAS operating in the band 10.6-10.7 GHz and other services | 2002 |
|  | ECC Report 003 | Fixed service in Europe current use and future trends POST-2002 | 2002 |
| 24.5 - 26.5 GHz 27.5 - 29.5 GHz | ERC Report 099 | Coexistence of two FWA cells in the 24.5 - 26.5 GHz and 27.5 - 29.5 GHz bands | 2000 |
|  | ERC Report 097 | Fixed Wireless Access (FWA) spectrum engineering | 2000 |
|  | ERC Report 068 | Monte-Carlo Radio Simulation Methodology | 2002 |
| 2 GHz | ERC Report 055 | Unwanted emission interference from mobile earth stations into fixed service receivers in the 2 GHz band | 1997 |
|  | ERC Report 051 | Sharing between military and civil radio services | 1997 |
| 55.22 - 55.78 GHz | ERC Report 046 | Sharing fixed service and Earth exploration-satellite service in 55.22 - 55.78 GHz | 1997 |
| 50.2 - 66 GHz | ERC Report 045 | Sharing Fixed and Earth Exploration Satellite (passive) Services in 50.2 - 66 GHz | 1997 |
|  | ERC Report 040 | Fixed service system parameters for frequency sharing | 1997 |
| 14.25 - 14.5 GHz | ERC Report 039 | Sharing between fixed links and SNG in the 14.25 - 14.5 GHz band | 1995 |
|  | ERC Report 036 | Sharing Fixed Service and Radio Astronomy | 1995 |
| above 20 GHz | ERC Report 033 | The use of frequencies above 20 GHz by fixed services and ENG/OB | 1994 |
|  | ERC Report 025 | European Common Allocation Table (ECA) | 2015 |
| 54.25-57.2 GHz | ERC Report 019 | Sharing Earth Exploration satellite services (passive) and Fixed Services in the band 54.25 - 57.2 GHz | 1993 |
| 38 GHz | ERC Report 016 | Sharing terrestrial fixed service and space research/EES (S - E) at 38 GHz | 1992 |
| 38 GHz | ERC Report 006 | Compatibility between RLANs and the Fixed Service | 1991 |

3. Reference lists of relevant ETSI standards for equipment characteristics for FS

3.1. Standards for P-P FS systems

Standard for P-P systems, including antennas, cover a very large range of traffic capacities, channel separations (CS), modulation formats and applications over a very wide range of frequency bands that are summarized in following table.

Digital Fixed Radio Systems (DFRS) parameters:

| Parameter | Frequency range: |
| --- | --- |
| Frequency bands | from 1 GHz to 86 GHz (100 GHz) |
| Traffic capacities | from 9.6 kbit/s to 622 Mbit/s and to Gigabit/s and above in the highest bands |
| Channel separations | from 25 kHz to 112 MHz and to Gigahertz and above in the highest bands |
| Modulation formats | from 2 to 512 states (amplitude and/or phase and/or frequency modulated states) |
| Typical applications | P-P CONNECTIONS: rural and urban low/medium/high capacity links for mobile infrastructure, transport/trunk (long haul), FWA/BWA/MWA backhaul, access, governmental (non-military) links, private fixed networks, SAP/SAB P to P audio and video links STAND ALONE ANTENNAS: for all of the above applications when integral antennas are not employed |

Generic standard for P-P digital fixed radio systems and antennas: EN 302 217.

EN 302 217 is a multipart standard including harmonised parts covering the essential requirements under article 3.2 of the 1999/5/EC Directive as described in the following Table.

EN 302 217 Multipart standard description

| Part | Subject | Status |
| --- | --- | --- |
| EN 302 217-1 | Overview and system-independent common characteristics | Not harmonised |
| EN 302 217-2-1 | System-dependent requirements for digital systems operating in frequency bands where frequency co-ordination is applied | Not harmonised |
| EN 302 217-2-2 | Digital systems operating in frequency bands where frequency co-ordination is applied; Harmonised EN covering the essential requirements of Article 3.2 of the R&TTE Directive. | Harmonised |
| EN 302 217-3 | Equipment operating in frequency bands where both frequency coordinated or uncoordinated deployment might be applied; Harmonised EN covering the essential requirements of Article 3.2 of the R&TTE Directive. | Harmonised |
| EN 302 217-4-1 | System-dependent requirements for antennas | Not-harmonised |
| EN 302 217-4-2 | Antennas; Harmonised EN covering the essential requirements of Article 3.2 of the R&TTE Directive | Harmonised |

3.2.Standards for Multipoint FS systems:

Standards for Multipoint systems, including antennas, cover a very large range of traffic capacities, channel separations (CS), modulation formats and applications over a very wide range of frequency bands:  from 30 MHz to 43.5 GHz for equipment and from 1 GHz to 43.5 GHz for antennas.

With the exception of systems and antennas dedicated to the band 40.5-43.5 GHz all other equipment operating in bands from 30 MHz to 33.4 GHz and antennas are specified in the generic standard for Multipoint digital fixed radio systems and antennas: EN 302 326.

The scope of this standard also includes Multipoint digital nomadic radio systems.

EN 302 326 is a multipart standard including harmonised parts covering the essential requirements under Article 3.2 of the Directive 1999/5/EC as described in the following Table.

EN 302 326 Multipart standard description:

| Part | Subject | Status |
| --- | --- | --- |
| EN 302 326-1 | Overview and requirements for Digital Multipoint Radio systems | Not harmonised |
| EN 302 326-2 | Harmonised EN covering the essential requirements of Article 3.2 of the R&TTE Directive for Digital Multipoint Radio Equipment | Harmonised |
| EN 302 326-3 | Harmonised EN covering the essential requirements of Article 3.2 of the R&TTE Directive for Multipoint Radio Antennas | Harmonised |

EN 302 326 supersedes former harmonised standard EN 301 753 as well as former system specific and antenna specific standards for P-MP systems.

By exception P-MP equipments operating in the frequency range 40.5-43.5 GHz are subject to a specific harmonised standard: EN 301 997, as explained in the following Table.

EN 301 997 description

| Part | Subject | Status |
| --- | --- | --- |
| EN 301 997-1 | Multipoint equipment; Radio Equipment for use in Multimedia Wireless Systems (MWS) in the frequency band 40.5 to 43.5 GHz; Part 1: General requirements | Not harmonised |
| EN 301 997-2 | Multipoint equipment; Radio Equipment for use in Multimedia Wireless Systems (MWS) in the frequency band 40.5 to 43.5 GHz; Part 2: Harmonized EN covering essential requirements under article 3.2 of the R&TTE Directive | Harmonised |

Note: this EN refers to EN 301 125-1 and EN 301 125-3 for directional parameters

As a consequence specific parts of antenna specific standard EN 301 215 remain valid for the frequency band 40.5-43.5 GHz as listed in the table below.

EN 301 215 description:

| Part | Subject |
| --- | --- |
| EN 301 215-1 | Fixed Radio Systems; Point to Multipoint Antennas; Antennas for P-MP fixed radio systems in the 11 GHz to 60 GHz band; Part 1: General aspects |
| EN 301 215-3 | Fixed Radio Systems; Point to Multipoint Antennas; Antennas for P-MP fixed radio systems in the 11 GHz to 60 GHz band; Part 3: Multipoint Multimedia Wireless System in 40.5 GHz to 43.5 GHz |

- 3.3. System specific standards

In addition to the technology neutral Harmonised Standards for Fixed P-MP systems ETSI has also developed Technical Specifications dedicated to the specific Fixed P-MP technologies HIPERACESS and HiperMAN.

HIPERACCESS system is designed to operate typically in frequency bands designated for P-MP use typically between 11 and 42 GHz.

HiperMAN  system is designed to operate in frequency bands designated for P-MP use below 11 GHz.