Appendix 1 to Annex 1.2.

HCM table of frequency allocations and utilisations (1 GHz to 100 GHz)

| Frequency bands: Radiocommunication services: RR(2012) and ECA(2015) | Proposals in relation to HCM Agreement: | Applications: Typical capacity and trends: |
| --- | --- | --- |
| 1350-1400 MHz FIXED MOBILE | HCM-Agreement (Art. 1.2.3) | Fixed (Low capacity fixed) |
| 1427-1525 MHz FIXED MOBILE exc. aer. mob. ADD 5.R1a, ADD 5.R2a, ADD 5.R3g MOD 5.338A 5.341 | HCM-Agreement (Art. 1.2.3) Extension of freq. band: 1452-1492 MHz (Art. 1.2.1) | Fixed (Low capacity fixed) |
| 1668.4-1690 MHz FIXED MOBILE exc. aer. mob. | Extension of HCMA (new freq. band) | Defence systems |
| 1710-1785 MHz FIXED MOBILE | HCM-Agreement (Art. 1.2.1) | GSM 1800 systems |
| 1785-1805 MHz FIXED MOBILE | Extension of HCMA (new freq. band) Preferential Agreements (to add to Art. 1.2.1 or Art 1.2.3 – different views) | Fixed (PtP) Land mobile Flex band |
| 1805-1880 MHz FIXED MOBILE | HCM-Agreement (Art. 1.2.1) | GSM 1800 systems LTE, IMT 1800 systems |
| 1900-1980 MHz FIXED MOBILE | HCM-Agreement (Art. 1.2.1) | UMTS/IMT-2000 ter.-systems |
| 1980-2010 MHz FIXED MOBILE | Extension of HCMA (new freq. band) | (MSS) |
| 2010-2025 MHz FIXED MOBILE | HCM-Agreement (Art. 1.2.1) | UMTS/IMT-2000 ter.-systems |
| 2025-2110 MHz FIXED MOBILE | HCM-Agreement (Art. 1.2.3) | Defence systems |
| 2110-2170 MHz FIXED MOBILE | HCM-Agreement (Art. 1.2.1) | UMTS/IMT-2000 ter.-systems |
| 2170-2200 MHz FIXED MOBILE | Extension of HCMA (new freq. band) | (MSS) |
| 2200-2290 MHz (ext.to 2300) FIXED MOBILE | HCM-Agreement (Art. 1.2.3) Extension of frequency band: 2290-2300 MHz | Defence systems (2200-2245 MHz) MFCN applications (PmP) |
| 2300-2450 MHz FIXED MOBILE | Extension of HCMA (new freq. band) (to add 2300 – 2400 MHz to Art. 1.2.1 and 2400-2450 MHz to Art 1.2.3 – different views) | MFCN applications Low/Medium capacity links |
| 2450-2500 MHz FIXED MOBILE | Extension of HCMA (new freq. band) | MFCN applications |
| 2500-2690 MHz FIXED MOBILE exc. aer. mob | HCM-Agreement (Art. 1.2.1) Bi- and Multilateral Agreements | Mobile applications LTE systems |
| 3400-3600 MHz FIXED Mobile (RR)/MOBILE (ECA) MOD 5.430A | HCM-Agreement (Art. 1.2.3) Bi- and Multilateral Agreement To add in (Art.1.2.1) in the future? New Bi-Multilateral Agreements in the future for MCFN | BWA – decrease of PtP Planned for MFCN applications in some members |
| 3600-3800 MHz FIXED Mobile (RR)/MOBILE (ECA) | HCM-Agreement (Art. 1.2.3) To add in (Art.1.2.1) in the future? New Bi-Multilateral Agreements in the future for MCFN | BWA Planned for MFCN applications in some members |
| 3800-4200 MHz FIXED Mobile (RR)/MOBILE (ECA) | HCM-Agreement (Art. 1.2.3) | BWA Fixed Low/Medium capacity |
| 4400-5000 MHz FIXED MOBILE | Extension of HCMA (new freq. band) | Defence systems |
| 5850-5925 MHz FIXED MOBILE | Extension of HCMA (new freq. band) | BFWA (civil networks) |
| 5925-6425 MHz FIXED MOBILE | Extension of HCMA (new service) HCM-Agreement (Art. 1.2.3) | Fixed (PtP) |
| 6425-8025 MHz FIXED MOBILE | Extension of HCMA (new service) HCM-Agreement (Art. 1.2.3) Extension of freq. band: 7975-8025 MHz | Fixed (PtP) Low/medium/high capacity |
| 8025-8400 MHz FIXED MOBILE | Extension of HCMA (new service) HCM-Agreement (Art. 1.2.3) | Fixed Land mobile |
| 8400-8500 MHz FIXED MOBILE exc. aer. mob. | Extension of HCMA (new service) HCM-Agreement (Art. 1.2.3) | Fixed (PtP) |
| 10-10.45 GHz FIXED MOBILE | Extension of HCMA (new service) HCM-Agreement (Art. 1.2.3) Extension of freq. band: 10-10.15 GHz | Fixed (PtP, PmP) BFWA Expected growth of FWA |
| 10.5-10.68 GHz FIXED MOBILE/ MOBILE exc. aer. mob. | Extension of HCMA (new service) HCM-Agreement (Art. 1.2.3) Extension of freq. band: 10.65-10.68 GHz | MVDS (uplink, PmP) Fixed BFWA Expected growth of FWA |
| 10.7-11.7 GHz FIXED MOBILE exc. aer. mob. | Extension of HCMA (new service) HCM-Agreement (Art. 1.2.3) | Fixed High capacity fixed links |
| 11.7-12.5 GHz FIXED MOBILE exc. aer. mob. | Extension of HCMA (new freq. band) HCM-Agreement (Art. 1.2.3) | MVDS (PmP) |
| 12.75-13.25 GHz FIXED MOBILE | Extension of HCMA (new service) HCM-Agreement (Art. 1.2.3) | Fixed Medium/High capacity links |
| 14.3-14.5 GHz FIXED MOBILE exc. aer. mob. | Extension of HCMA (new service) HCM-Agreement (Art. 1.2.3) | Fixed links |
| 14.5-15.35 GHz FIXED MOBILE | Extension of frequency band: 14.62-15.23 GHz HCM-Agreement (Art. 1.2.3) | Defence systems Fixed links |
| 17.7-19.7 GHz FIXED MOBILE (RR) | HCM-Agreement (Art. 1.2.3) | Fixed links Low/Medium/High capacity links Priority for civil networks |
| 21.2-22 GHz FIXED MOBILE | Extension of HCMA (new freq. band) HCM-Agreement (Art. 1.2.3) |  |
| 22-23.6 GHz FIXED MOBILE | Extension of HCMA (new service) HCM-Agreement (Art. 1.2.3) | Fixed links (PtP) |
| 24.25-24.45 GHz FIXED MOBILE (ECA) | Extension of frequency band HCM-Agreement (Art. 1.2.3) | Fixed links (PtP) |
| 24.45-27.5 GHz FIXED MOBILE | Extension of frequency band: 24.45-24.5 GHz and 26.5-27.5 GHz HCM-Agreement (Art. 1.2.3) Bi- and Multilateral Agreements | Fixed BFWA |
| 27.5-29.5 GHz FIXED MOBILE (RR) | HCM-Agreement (Art. 1.2.3) Bi- and Multilateral Agreements | Fixed (PtP, PmP) BFWA ECC/DEC/(05)01 |
| 31-31.3 GHz FIXED MOBILE | Extension of HCMA (new freq. band) HCM-Agreement (Art. 1.2.3) | Fixed |
| 31.8-33.4 GHz FIXED | HCM-Agreement (Art. 1.2.3) | Fixed (PtP, PmP) High density FS |
| 36-37 GHz FIXED MOBILE | Extension of HCMA (new freq. band) HCM-Agreement (Art. 1.2.3) | Defence systems |
| 37-39.5 GHz FIXED MOBILE exc. aer. mobile | HCM-Agreement (Art. 1.2.3) | Defence systems Fixed (PtP) L/M/High capacity links |
| 39.5-40.5 GHz FIXED MOBILE | Extension of HCMA (new freq. band) HCM-Agreement (Art. 1.2.3) |  |
| 40.5-43.5 GHz FIXED | HCM-Agreement (Art. 1.2.3) | Fixed (PtP, PmP) MWS |
| 43.5-47 GHz MOBILE | Extension of HCMA (new freq. band) HCM-Agreement (Art. 1.2.1) | Defence systems |
| 47.2-50.2 GHz FIXED MOBILE | Extension of HCMA (new freq. band) HCM-Agreement (Art. 1.2.3) | Fixed links (Light licensing/individual licensing) |
| 50.4-52.6 GHz FIXED MOBILE (RR) | Extension of HCMA (new freq. band) HCM-Agreement (Art. 1.2.3) | Defence systems Fixed High density fixed links (Light licensing/individual licensing) |
| 55.78-64 GHz FIXED MOBILE (RR) | Extension of HCMA (new freq. band) HCM-Agreement (Art. 1.2.3) | Fixed High density fixed links PtP (General licensing)/light licensing/individual licensing) |
| 64-66 GHz FIXED MOBILE exc. aer. mob. | Extension of HCMA (new freq. band) HCM-Agreement (Art. 1.2.3) | Fixed High density fixed links PtP (General licensing)/light licensing/individual licensing) |
| 66-71 GHz MOBILE | Extension of HCMA (new freq. band) HCM-Agreement (Art. 1.2.3) |  |
| 71-76 GHz FIXED MOBILE | Extension of HCMA (new freq. band) HCM-Agreement (Art. 1.2.3) | Fixed High density fixed links PtP (General licensing)/light licensing |
| 81-86 GHz FIXED MOBILE | Extension of HCMA (new freq. band) HCM-Agreement (Art. 1.2.3) | Fixed PtP (General licensing) )/light licensing |
| 92-94 GHz FIXED MOBILE | Extension of HCMA (new freq. band) HCM-Agreement (Art. 1.2.3) | Fixed (General licensing)/light licensing/individual licensing) |
| 94.1-100 GHz FIXED MOBILE |  |  |