## Appendix 1 to Annex 2.2.

## Table 2.1: Minimum equipment requirements

| Frequency range (MHz) | Channel plan | Equipment reference code |  |  | Minimum bit rate (Mbits/s) | Channel spacing (MHz) | Maximum output power (dBm) | Minimum link length (km) | Reference standards assumed to be fulfilled in frequency planning and defining the equipment type | Frequency Assignment Criteria | Additional information |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Part A | Part B | Part C |  |  |  |  |  |  |  |
| 1350-1517 MHz FIXED MOBILE | 1350 MHz to 1375 MHz paired with 1492 MHz to 1517 MHz with 142 MHz duplexing in accordance with CEPT Recommendation T/R 13-01 | 9k6 | 25k | - | 9.6 kbits/s | 25 kHz | +40 | N/A | EN 300 630 V1.3.1 (February 2001) Fixed Radio Systems; point-to-point equipment; Low capacity point-to-point digital radio systems operating in the 1.4 GHz frequency band | RA 349 |  |
|  |  | 64k | 75k | - | 64 kbits/s | 75 kHz | +40 | N/A |  |  |  |
|  |  | 120k | 250k | - | 120 kbits/s | 250 kHz | +40 | N/A |  |  |  |
|  |  | 384k | 500k | - | 384 kbits/s | 500 kHz | +40 | N/A |  |  |  |
|  |  | 2M | 1M | - | 2.048 | 1 | +40 | 30 |  |  |  |
|  |  | 4M | 2M | - | 4.096 | 2 | +40 | 30 |  |  |  |
| 3400-4200 MHz FIXED Mobile (RR)/ MOBILE (ECA) | The Channel plan is in accordance with CEPT/ERC Recommendation 12-08 E | 34M | 15M | - | 34 | 15 | +40 | 30 | EN 301 216 V1.1.1 (September 1999) Transmission and Multiplexing (TM; Digital Radio Relay Systems (DRRS); Plesiochronous Digital Hierarchy (PDH); Low and medium capacity and DTM-0 DRRS operating in the frequency bands in the range 3 GHz to 11 GHz | RA 353 | Mobile (RR)/MOBILE |
|  |  | 68M | 15M | - | 2 x 34 | 15 | +40 | 30 |  |  |  |
|  |  | 51M | 15M | - | 51 | 15 | +40 | 30 |  |  |  |
|  |  | 140M | 30M | - | 140/155 | 30 | +38 | 16 | EN 300 234 V1.3.1 (February 2001) Fixed Radio Systems; Point-to-point equipment; High capacity digital radio systems carrying 1 x STM-1 signals and operating in frequency bands with about 30 MHz channel spacing and alternated arrangements |  |  |
| 5925-6425 MHz FIXED MOBILE | The Channel plan is in accordance with CEPT/ERC Recommendation 14-01 E | 140M | 29M6 | - | 140/155 | 29.65 | +38 | 16 | EN 300 234 V1.3.1 (February 2001) Fixed Radio Systems; Point-to-point equipment; High capacity digital radio systems carrying 1 x STM-1 signals and operating in frequency bands with about 30 MHz channel spacing and alternated arrangements | RA 374 |  |
|  |  | 280M | 29M6 | - | 280/310 | 29.65 | +38 | 16 | EN 301 127 V1.1.1 (September 1999) Transmission and Multiplexing (TM); Digital Radio Relay Systems (DRRS); Synchronous Digital Hierarchy (SDH); High capacity DRRS carrying SDH signals (2 x STM-1) in frequency bands with about 30 MHz channel spacing and using Co-Channel Dual Polarised (CCDP) operation. |  |  |

| Frequency range (MHz) | Channel plan | Equipment reference code |  |  | Minimum bit rate (Mbits/s) | Channel spacing (MHz) | Maximum output power (dBm) | Minimum link length (km) | Reference standards assumed to be fulfilled in frequency planning and defining the equipment type | Frequency Assignment Criteria | Additional information |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Part A | Part B | Part C |  |  |  |  |  |  |  |
| 6425-7425 MHz FIXED MOBILE | The Channel plan is in accordance with CEPT/ERC Recommendation 14-02 E | 280M | 40M | - | 280/310 | 40 | +38 | 16 | EN 301 461 V1.2.1 (February 2001) Fixed Radio Systems; Point-to-point equipment; High capacity fixed radio systems carrying SDH signals (2 x STM-1) in frequency bands with 40 MHz channel spacing and using Co-channel Dual Polarized (CCDP) operation |  |  |
|  |  | 280M | 40M | - | 280/310 | 40 | +37 | 16 | EN 301 669 V1.2.1 (February 2001) Fixed Radio Systems; Point-to-point equipment; High capacity digital radio systems carrying STM-4 in two 40 MHz channels or 2 x STM-1 in a 40 MHz channel with alternate channel arrangement |  |  |
|  |  | 560M | 40M | - | 560/620 | 40 | +37 | 16 |  |  |  |
| 7425-7900 MHz FIXED MOBILE | The Channel plan is in accordance with Recommendation ITU-R 385-6, Annex 4 | 8M | 7M | - | 8 | 7 | +40 | 30 | EN 301 216 V1.1.1 (September 1999) Transmission and Multiplexing (TM; Digital Radio Relay Systems (DRRS); Plesiochronous Digital Hierarchy (PDH); Low and medium capacity and STM-0 DRRS operating in the frequency bands in the range 3 GHz to 11 GHz |  |  |
|  |  | 16M | 7M | - | 16 | 7 | +40 | 30 |  |  |  |
|  |  | 34M | 14M | - | 34 | 14 | +40 | 30 |  |  |  |
|  |  | 51M | 14M | - | 51 | 14 | +40 | 30 |  |  |  |
|  |  | 68M | 14M | - | 2 x 34 | 14 | +40 | 30 |  |  |  |
|  |  | 68M | 28M | - | 2 x 34 | 28 | +40 | 30 |  |  |  |
|  |  | 140M | 28M | - | 140/155 | 28 | +38 | 12.5 | EN 300 234 V1.3.1 (February 2001) Fixed Radio Systems; Point-to-point equipment; High capacity digital radio systems carrying 1 x STM-1 signals and operating in frequency bands with about 30 MHz channel spacing and alternated arrangements |  |  |
|  |  | 10T | 28M | - | < 10 (Analogue) | 28 | See Additional Information | 30 | VNS to be drafted by RA and notified under 98/34/EC |  | T = FM-Video Trunk: Analogue fm-video (10 or 14 MHz baseband) in 28 MHz channel spacing Power Limits: The carrier output power under extreme test conditions shall be within ±2dB of manufactures maximum rated output power, and shall not exceed ITU Radio Regulations Article S 21 limits, where applicable. |
|  |  | 14T | 28M | - | < 14 (Analogue) | 28 |  | 30 |  |  |  |
| 10.7-11.7 GHz FIXED MOBILE (exc. aer. mob.) | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |  | Allocated exclusively to and managed by a Public Telecommunications operator. It is intended that in the future the RA will take back the management of this band. |

| Frequency range (MHz) | Channel plan | Equipment reference code |  |  | Minimum bit rate (Mbits/s) | Channel spacing (MHz) | Maximum output power (dBm) | Minimum link length (km) | Reference standards assumed to be fulfilled in frequency planning and defining the equipment type | Frequency Assignment Criteria | Additional information |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Part A | Part B | Part C |  |  |  |  |  |  |  |
| 12.75-13.25 GHz FIXED MOBILE | The Channel plan is in accordance with CEPT/ERC Recommendation 12-02 E | 2M | 1M7 | - | 2 | 1.75 | +40 | 15 | EN 301 128 V1.2.1 (February 2001) Fixed Radio Systems; Point-to-point equipment; Plesiochronous Digital Hierarchy (PDH); Low and medium capacity digital radio systems operating in the 13 GHz, 15 GHz and 18 GHz frequency bands | MPT 1403 Part 4 |  |
|  |  | 4M | 1M7 | - | 2x2 | 1.75 | +40 | 15 |  |  |  |
|  |  | 4M | 3M5 | - | 2x2 | 3.5 | +40 | 15 |  |  |  |
|  |  | 8M | 3M5 | - | 8 | 3.5 | +40 | 15 |  |  |  |
|  |  | 8M | 7M | - | 8 | 7 | +40 | 15 |  |  |  |
|  |  | 16M | 7M | - | 2x8 | 7 | +40 | 15 |  |  |  |
|  |  | 16M | 14M | - | 2x8 | 14 | +40 | 15 |  |  |  |
|  |  | 34M | 14M | - | 34 | 14 | +40 | 15 |  |  |  |
|  |  | 34M | 28M | - | 34 | 28 | +40 | 15 |  |  |  |
|  |  | 68M | 28M | - | 2x34 | 28 | +40 | 15 |  |  |  |
|  |  | 51M | 28M |  | 51 | 28 | +30 | 15 | EN 300 639 V1.3.1 (February 2001) Fixed Radio Systems; Point-to-point equipment; Sub-STM -1 digital radio systems operating in the 13 GHz, 15 GHz and 18 GHz frequency bands with about 28 MHz co-polar and 14 MHz cross-polar channel spacing |  |  |
|  |  | 140M | 28M | - | 140/155 | 28 | +38 | 6 | EN 300 234 V1.3.1 (February 2001) Fixed Radio Systems; Point-to-point equipment; High capacity digital radio systems carrying 1 x STM-1 signals and operating in frequency bands with about 30 MHz channel spacing and alternated arrangements |  |  |
|  |  | 10T | 28M | - | < 10 (Analogue) | 28 | See Additional Information | 15 | VNS to be drafted by RA and notified under 98/34/EC |  | T = FM-Video Trunk: Analogue fm-video (10 or 14 MHz baseband) in 28 MHz channel spacing Power Limits: The carrier output power under extreme test conditions shall be within ±2dB of manufactures maximum rated output power, and shall not exceed ITU Radio Regulations Article S 21 limits, where applicable. |
|  |  | 14T | 28M | - | < 14 (Analogue) | 28 |  | 15 |  |  |  |

| Frequency range (MHz) | Channel plan | Equipment reference code |  |  | Minimum bit rate (Mbits/s) | Channel spacing (MHz) | Maximum output power (dBm) | Minimum link length (km) | Reference standards assumed to be fulfilled in frequency planning and defining the equipment type | Frequency Assignment Criteria | Additional information |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Part A | Part B | Part C |  |  |  |  |  |  |  |  |
| 14.25-14.5 GHz FIXED MOBILE | The Channel plan is in accordance with Recommendation ITU-R 746-3, Annex 5 | 2M | 1M7 | - | 2 | 1.75 | +40 | 15 | UK Voluntary National Specification 2103 Performance parameters for fixed (point-to-point) terrestrial equipment operating in the 14.25 GHz to 14.5 GHz band (Version 1.0). | MPT 1403 Part 4 |  |  |
|  |  | 4M | 1M7 | - | 2x2 | 1.75 | +40 | 15 |  |  |  |  |
|  |  | 4M | 3M5 | - | 2x2 | 3.5 | +40 | 15 |  |  |  |  |
|  |  | 8M | 3M5 | - | 8 | 3.5 | +40 | 15 |  |  |  |  |
|  |  | 8M | 7M | - | 8 | 7 | +40 | 15 |  |  |  |  |
|  |  | 16M | 7M | - | 2x8 | 7 | +40 | 15 |  |  |  |  |
|  |  | 16M | 14M | - | 2x8 | 14 | +40 | 15 |  |  |  |  |
|  |  | 34M | 14M | - | 34 | 14 | +40 | 15 |  |  |  |  |
|  |  | 34M | 28M | - | 34 | 28 | +40 | 15 |  |  |  |  |
|  |  | 68M | 28M | - | 2x34 | 28 | +40 | 15 |  |  |  |  |
|  |  | 140M | 28M | - | 140/155 | 28 | +38 | 6 |  |  |  |  |
|  |  | 10T | 28M | - | < 10 (Analogue) | 28 | See Additional Information | 15 | VNS to be drafted by RA and notified under 98/34/EC |  | T = FM-Video Trunk: Analogue fm-video (10 or 14 MHz baseband) in 28 MHz channel spacing Power Limits: The carrier output power under extreme test conditions shall be within ±2dB of manufactures maximum rated output power, and shall not exceed ITU Radio Regulations Article S 21 limits, where applicable. |  |
|  |  | 14T | 28M | - | < 14 (Analogue) | 28 |  | 15 |  |  |  |  |
| 14.25-14.5 GHz FIXED MOBILE | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | Allocated exclusively to and managed by a Public Telecommunications operator. It is intended that in the future the RA will take back the management of this band. |  |
| 17.7-17.7 GHz FIXED MOBILE | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | Allocated exclusively to and managed by a Public Telecommunications operator. It is intended that in the future the RA will take back the management of this band. |  |

| Frequency range (MHz) | Channel plan | Equipment reference code |  |  | Minimum bit rate (Mbits/s) | Channel spacing (MHz) | Maximum output power (dBm) | Minimum link length (km) | Reference standards assumed to be fulfilled in frequency planning and defining the equipment type | Frequency Assignment Criteria | Additional information |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Part A | Part B | Part C |  |  |  |  |  |  |  |
| 22-23.6 GHz FIXED MOBILE | The Channel plan is in accordance with CEPT Recommendation T/R 13-02. | 2M | 3M5 | - | 2 | 3.5 | +30 | 5 | EN 300 198 V1.4.1 (February 2001) Fixed Radio Systems; Point-to-Point equipment; Parameters for radio systems for the transmission of digital signals operating at 23 GHz | RA 352 |  |
|  |  | 4M | 3M5 | - | 2 x 2 | 3.5 | +30 | 5 |  |  |  |
|  |  | 8M | 7M | - | 8 | 7 | +30 | 5 |  |  |  |
|  |  | 16M | 14M | - | 2 x 8 | 14 | +30 | 5 |  |  |  |
|  |  | 34M | 28M | - | 34 | 28 | +30 | 5 |  |  |  |
|  |  | 8M | 3M5 | - | 8 | 3.5 | +30 | 5 |  |  |  |
|  |  | 16M | 7M | - | 2 x 8 | 7 | +30 | 5 |  |  |  |
|  |  | 34M | 14M | - | 34 | 14 | +30 | 5 |  |  |  |
|  |  | 51M | 14M | - | 51 | 14 | +30 | 5 |  |  |  |
|  |  | 51M | 28M | - | 51 | 28 | +30 | 5 |  |  |  |
|  |  | 140M1 | 28M | A | 140/155 | 28 | +30 | 2.5 |  |  |  |
|  |  | 140M2 | 28M | B | 140/155 | 28 | +30 | 2.5 |  |  |  |
|  |  | 140M | 56M | - | 140/155 | 56 | +30 | 2.5 |  |  |  |
|  |  | 280M | 56M | - | 280/310 | 56 | +30 | 2.5 | DEN/TM 4105 (June 2000) Fixed Radio systems; Point-to-point equipment; High capacity digital radio relay systems carrying 2 x STM-1, 4 x STM-1 or STM-4 signals in bands with 55/56 MHz CS |  |  |
|  |  | 3S5 | 21M | - | < 3.5 (Analogue) | 21 | +30 See Additional Information | 5 |  | RA 351 | S = FM-Video Surveillance: Analogue fm-video (3.5 MHz Baseband) in 21 or 42 MHz channel spacings. T = FM-Video Trunk: Analogue fm-video (10 or 14 MHz Baseband) in 28 MHz channel spacing Power Limits: The carrier output power under extreme test conditions shall be within ±2dB of manufactures maximum rated output power, and shall not exceed ITU Radio Regulations Article S 21 limits, where applicable. |
|  |  | 10S | 42M | - | <10 (Analogue) | 42 |  | 5 |  |  |  |
|  |  | 10T | 28M | - | < 10 (Analogue) | 28 |  | 5 |  |  |  |
|  |  | 14T | 28M | - | < 14 (Analogue) | 28 |  | 5 |  |  |  |

| Frequency range (MHz) | Channel plan | Equipment reference code |  |  | Minimum bit rate (Mbits/s) | Channel spacing (MHz) | Maximum output power (dBm) | Minimum link length (km) | Reference standards assumed to be fulfilled in frequency planning and defining the equipment type | Frequency Assignment Criteria | Additional information |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Part A | Part B | Part C |  |  |  |  |  |  |  |
| 24.45-27.5 GHz FIXED MOBILE | The Channel plan is in accordance with CEPT Recommendation T/R 13-02E Annex B | 2M | 3M5 | - | 2 | 3.5 | +30 | 5 | EN 300 431 V1.3.1 (February 2001) Fixed Radio Systems; Point-to-point equipment; Parameters for radio system for the transmission of digital signals operating in the frequency range 24,5 GHz to 29,5 GHz | RA 348 | Channel allocation will depend upon system bandwidth. For further details contact the RA. |
|  |  | 4M | 3M5 | - | 2 x 2 | 3.5 | +30 | 5 |  |  |  |
|  |  | 8M | 3M5 | - | 8 | 3.5 | +30 | 5 |  |  |  |
|  |  | 8M | 7M | - | 8 | 7 | +30 | 5 |  |  |  |
|  |  | 16M | 7M | - | 2 x 8 | 7 | +30 | 5 |  |  |  |
|  |  | 16M | 14M | - | 2 x 8 | 14 | +30 | 5 |  |  |  |
|  |  | 34M | 14M | - | 34 | 14 | +30 | 5 |  |  |  |
|  |  | 34M | 28M | - | 34 | 28 | +30 | 5 |  |  |  |
|  |  | 51M | 14M | - | 51 | 14 | +30 | 5 |  |  |  |
|  |  | 51M | 28M | - | 51 | 28 | +30 | 5 |  |  |  |
|  |  | 140M1 | 28M | A | 140/155 | 28 | +30 | 2.5 |  |  |  |
|  |  | 140M2 | 28M | B | 140/155 | 28 | +30 | 2.5 |  |  |  |
|  |  | 140M | 56M | - | 140/155 | 56 | +30 | 2.5 |  |  |  |
|  |  | 280M | 56M | - | 280/310 | 56 | +30 | 2.5 | DEN/TM 4105 (June 2000) Fixed Radio systems; Point-to-point equipment; High capacity digital radio relay systems carrying 2 x STM-1, 4 x STM-1 or STM-4 signals in bands with 55/56 MHz channel spacing |  |  |
| 31-31.8 GHz FIXED MOBILE | National Channel plan based on 28MHz raster. | FM | 28M | - | (Analogue) | 28 | +30 | N/A | UK Voluntary National Specification 2125 Performance parameters for fixed(point-to-point) terrestrial security equipment operating in the 31 GHz band (Version 1.0) | MPT 1425 Part 4 | FM: Analogue fm Surveillance in 28 MHz channel spacing |

| Frequency range (MHz) | Channel plan | Equipment reference code |  |  | Minimum bit rate (Mbits/s) | Channel spacing (MHz) | Maximum output power (dBm) | Minimum link length (km) | Reference standards assumed to be fulfilled in frequency planning and defining the equipment type | Frequency Assignment Criteria | Additional information |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Part A | Part B | Part C |  |  |  |  |  |  |  |
| 37-39.5 GHz FIXED MOBILE | The Channel plan is in accordance with CEPT Recommendation T/R 12-01 | 2M | 3M5 | - | 2 | 3.5 | +30 | 1 | EN 300 197 V1.3.1 (November 1999) Fixed Radio Systems; Point-to-Point Equipment; Parameters for radio systems for the transmission of digital signals operating at 38 GHz | RA 350 |  |
|  |  | 4M | 3M5 | - | 2 x 2 | 3.5 | +30 | 1 |  |  |  |
|  |  | 8M | 3M5 | - | 8 | 3.5 | +30 | 1 |  |  |  |
|  |  | 8M | 7M | - | 8 | 7 | +30 | 1 |  |  |  |
|  |  | 16M | 14M | - | 2 x 8 | 14 | +30 | 1 |  |  |  |
|  |  | 34M | 28M | - | 34 | 28 | +30 | 1 |  |  |  |
|  |  | 16M | 7M | - | 2 x 8 | 7 | +30 | 1 |  |  |  |
|  |  | 34M | 14M | - | 34 | 14 | +30 | 1 |  |  |  |
|  |  | 51M | 14M | - | 51 | 14 | +30 | 1 |  |  |  |
|  |  | 51M | 28M | - | 51 | 28 | +30 | 1 |  |  |  |
|  |  | 140M1 | 28M | A | 140/155 | 28 | +30 | 1 |  |  |  |
|  |  | 140M2 | 28M | B | 140/155 | 28 | +30 | 1 |  |  |  |
|  |  | 140M | 56M | - | 140/155 | 56 | +30 | 1 |  |  |  |
|  |  | 280M | 56M | - | 280/310 | 56 | +30 | 1 | DEN/TM 4105 (June 2000) FRS; Point-to-point equipment; High capacity digital radio relay systems carrying 2 x STM-1, 4 x STM-1 or STM-4 signals in bands with 55/56 MHz CS |  |  |
| 47.2-50.2 GHz FIXED MOBILE | The Channel plan is in accordance with CEPT/ERC Recommendation 12–10 E | 2M | 3M5 | - | 2 | 3.5 | +30 | N/A | EN 301 387 V1.2.1 (February 2001) Fixed Radio Systems; Point-to-point equipment; Plesiochronous Digital Hierarchy (PDH); Low and medium capacity digital radio systems operating in the frequency band 48.5 GHz to 50.2 GHz |  |  |
|  |  | 4M | 3M5 | - | 4 | 3.5 | +30 | N/A |  |  |  |
|  |  | 8M | 7M | - | 8 | 7 | +30 | N/A |  |  |  |
|  |  | 16M | 14M | - | 16 | 14 | +30 | N/A |  |  |  |
|  |  | 34M | 28M | - | 34 | 28 | +30 | N/A |  |  |  |
| 50.4-52.6 GHz FIXED MOBILE | The Channel plan is in accordance with CEPT/ERC Recommendation 12–11 E | 16M | 14M | - | 16 | 14 | +10 | N/A | EN 301 786 V1.1.1 (April 2000) Fixed Radio Systems; Pont-to-point equipment; Parameters for digital radio systems for the transmission of digital signals operating at 52 GHz |  |  |
|  |  | 34M | 14M | - | 34 | 14 | +10 | N/A |  |  |  |
|  |  | 51M | 14M | - | 51 | 14 | +10 | N/A |  |  |  |
|  |  | 34M | 28M | - | 34 | 28 | +10 | N/A |  |  |  |
|  |  | 51M 140M | 28M 56M | - | 51 140/155 | 28 56 | +10 | N/A |  |  |  |
| Frequency range (MHz) | Channel plan | Equipment reference code |  |  | Minimum bit rate (Mbits/s) | Channel spacing (MHz) | Maximum output power (dBm) | Minimum link length (km) | Reference standards assumed to be fulfilled in frequency planning and defining the equipment type | Frequency Assignment Criteria | Additional information |
| 55.78-64 GHz FIXED MOBILE | The Channel plan is in accordance with CEPT/ERC Recommendation 12–12 E | 16M | 14M | - | 16 | 14 | +10 | N/A | EN 300 407 V1.2.1 (October 2000) Fixed Radio Systems; Point-to-point equipment; Parameters for digital radio systems for the transmission of digital signals operating at 55 GHz |  |  |
|  |  | 34M | 14M | - | 34 | 14 | +10 | N/A |  |  |  |
|  |  | 51M | 14M | - | 51 | 14 | +10 | N/A |  |  |  |
|  |  | 34M | 28M | - | 34 | 28 | +10 | N/A |  |  |  |
|  |  | 51M 140M | 28M 56M | - | 51 140/155 | 28 56 | +10 | N/A |  |  |  |
| 55.78-64 GHz FIXED MOBILE | The Channel plan is in accordance with CEPT/ERC Recommendation 12–12 E | 16M | 14M | - | 16 | 14 | +10 | N/A | EN 300 407 V1.2.1 (October 2000) Fixed Radio Systems; Point-to-point equipment; Parameters for digital radio systems for the transmission of digital signals operating at 55 GHz |  |  |
|  |  | 34M | 14M | - | 34 | 14 | +10 | N/A |  |  |  |
|  |  | 51M | 14M | - | 51 | 14 | +10 | N/A |  |  |  |
|  |  | 34M | 28M | - | 34 | 28 | +10 | N/A |  |  |  |
|  |  | 51M 140M | 28M 56M | - | 51 140/155 | 28 56 | +10 | N/A |  |  |  |

| Frequency range (MHz) | Channel plan | Equipment reference code |  |  | Minimum bit rate (Mbits/s) | Channel spacing (MHz) | Maximum output power (dBm) | Minimum link length (km) | Reference standards assumed to be fulfilled in frequency planning and defining the equipment type (digital equipment unless otherwise stated) | Frequency Assignment Criteria | Additional information |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Part A | Part B | Part C |  |  |  |  |  |  |  |
| 57-59 GHz FIXED MOBILE | The Channel plan is in accordance with CEPT/ERC Recommendation 12-09 | FM | 100M | - | (Analogue) | 100 | +10 | N/A | EN 300 408 V1.2.1 (September 2000) Fixed Radio Systems; Point-to-point equipment; Parameters for digital radio systems for the transmission of digital signals and analogue video signals at around 58 GHz, which do not require co-ordinated frequency planing | N/A | FM: Analogue fm Surveillance in 100 MHz channel spacing. Uncoordinated frequency band. |
|  |  | D | 50M | - | (Digital) | 50 | +10 | N/A |  |  | Digital system in 50 MHz and 100 MHz channel spacing. Uncoordinated frequency band. |
|  |  | D | 100M | - | (Digital) | 100 | +10 | N/A |  |  |  |

- This Licence Class is for Adjacent Channel Alternate-Polarisation operation only.

- This Licence Class is for Adjacent Channel Co-Polarisation operation only.

## Table 2.2: Minimum antenna requirements

| Frequency range (MHz) | Antenna reference codes | Radiation Pattern Envelope | Minimum cross polar discrimination (dB) | Minimum antenna gain (dBi) | Reference standards assumed to be fulfilled in frequency planning and defining the antenna type | Additional information |
| --- | --- | --- | --- | --- | --- | --- |
| 1350-1517 MHz FIXED MOBILE | 631/16 | Figure 1 (Azimuth) and Figure 6 (Elevation) in Reference Standard for CS 25, 75 and 250 kHz. | 20 | 13 | EN 300 631 V1.2.1 (December 1999) Fixed Radio Systems; Point-to-Point Antennas; Antennas for Point-to-Point fixed radio systems In the 1 GHz to 3 GHz band |  |
|  | 631/2 | Figure 2 in Reference Standard for CS of 25, 75, 250 and 500 kHz | 25 | 16 |  |  |
|  | 631/3 | Figure 3 in Reference Standard for channel spacing of 25, 75, 250 and 500 kHz |  |  |  |  |
|  | 631/4 | Figure 4 in Reference Standard for CS of 25, 75, 250 and 500 kHz | 25 | 20 |  |  |
|  | 631/5 | Figure 5 in Reference Standard for CS of 25, 75, 250, 500, 1000 and 2000 kHz |  |  |  |  |
| 3600-4200 MHz FIXED Mobile (RR)/ MOBILE (ECA) | 833/2c | See Figure 2c in the Reference Standard | 27 | 32 | EN 300 833 V1.2.1 (August 2000) Fixed Radio systems; Point to point Antennas; Antennas for point-to-point fixed radio systems operating in the frequency band 3 GHz to 60 GHz |  |
|  | 833/2d | See Figure 2d in the Reference Standard |  |  |  |  |
| 5925-6425 MHz FIXED MOBILE | 833/2c | See Figure 2c in the Reference Standard | 27 | 32 | EN 300 833 V1.2.1 (August 2000) Fixed Radio systems; Point to point Antennas; Antennas for point-to-point fixed radio systems operating in the frequency band 3 GHz to 60 GHz |  |
|  | 833/2d | See Figure 2d in the Reference Standard |  |  |  |  |
| 6425-7425 MHz FIXED MOBILE | 833/2c | See Figure 2c in the Reference Standard | 27 | 32 | EN 300 833 V1.2.1 (August 2000) Fixed Radio systems; Point to point Antennas; Antennas for point-to-point fixed radio systems operating in the frequency band 3 GHz to 60 GHz |  |
|  | 833/2d | See Figure 2d in the Reference Standard |  |  |  |  |
| 7425-7900 MHz FIXED MOBILE | 833/2c | See Figure 2c in the Reference Standard | 27 | 32 | EN 300 833 V1.2.1 (August 2000) Fixed Radio systems; Point to point Antennas; Antennas for point-to-point fixed radio systems operating in the frequency band 3 GHz to 60 GHz |  |
|  | 833/2d | See Figure 2d in the Reference Standard |  |  |  |  |
| 10.7-11.7 GHz FIXED MOBILE (exc. aer. mob.) | N/A | N/A | N/A | N/A | EN 300 833 V1.2.1 (August 2000) Fixed Radio systems; Point to point Antennas; Antennas for point-to-point fixed radio systems operating in the frequency band 3 GHz to 60 GHz |  |
| 12.75-13.25 GHz FIXED MOBILE | 833/2c | See Figure 2c in the Reference Standard | 27 | 32 | EN 300 833 V1.2.1 (August 2000) Fixed Radio systems; Point to point Antennas; Antennas for point-to-point fixed radio systems operating in the frequency band 3 GHz to 60 GHz |  |
|  | 833/2d | See Figure 2d in the Reference Standard |  |  |  |  |
| 14.25-14.5 GHz FIXED MOBILE | 833/3b | See Figure 3b in the Reference Standard | 27 | 32 | EN 300 833 V1.2.1 (August 2000) Fixed Radio systems; Point to point Antennas; Antennas for point-to-point fixed radio systems operating in the frequency band 3 GHz to 60 GHz |  |
|  | 833/3c | See Figure 3c in the Reference Standard |  |  |  |  |
| 14.5-15.35 GHz FIXED MOBILE | N/A | N/A | N/A | N/A | EN 300 833 V1.2.1 (August 2000) Fixed Radio systems; Point to point Antennas; Antennas for point-to-point fixed radio systems operating in the frequency band 3 GHz to 60 GHz | Allocated exclusively to and managed by Public Telecommunications operator. It is intended that in the future the RA will take back the management of this band. |
| 17.7-17.7 GHz FIXED MOBILE | N/A | N/A | N/A | N/A | EN 300 833 V1.2.1 (August 2000) Fixed Radio systems; Point to point Antennas; Antennas for point-to-point fixed radio systems operating in the frequency band 3 GHz to 60 GHz | Allocated exclusively to and managed by Public Telecommunications operator. It is intended that in the future the RA will take back the management of this band. |
| 22-23.6 GHz FIXED MOBILE | 833/4c | See Figure 4c in the Reference Standard | 27 | 32 | EN 300 833 V1.2.1 (August 2000) Fixed Radio systems; Point to point Antennas; Antennas for point-to-point fixed radio systems operating in the frequency band 3 GHz to 60 GHz |  |
| 24.45-27.5 GHz FIXED MOBILE | 833/5b | See Figure 5b in the Reference Standard | 27 | 32 | EN 300 833 V1.2.1 (August 2000) Fixed Radio systems; Point to point Antennas; Antennas for point-to-point fixed radio systems operating in the frequency band 3 GHz to 60 GHz |  |
| 31-31.8 GHz FIXED MOBILE | 833/6b | See Figure 6b in the Reference Standard | 27 | 32 | EN 300 833 V1.2.1 (August 2000) Fixed Radio systems; Point to point Antennas; Antennas for point-to-point fixed radio systems operating in the frequency band 3 GHz to 60 GHz |  |
|  |  |  |  |  | VNS 2125 is provided in the absence (or likelihood) of any Candidate Harmonised standard for the 31 GHz frequency band. VNS 2125 identifies what the Radiocommunications Agency considers the “essential parameters” and numerical values associated with these parameters, in conformance with the R&TTE Directive. The assertion of VNS 2125 is to give guidance to the notified bodies when assessing Technical Construction Files. |  |
| 37-39.5 GHz FIXED MOBILE | 833/6c | See Figure 6c in the Reference Standard | 27 | 32 | EN 300 833 V1.2.1 (August 2000) Fixed Radio systems; Point to point Antennas; Antennas for point-to-point fixed radio systems operating in the frequency band 3 GHz to 60 GHz |  |
| 48.54-50.2 GHz FIXED MOBILE | 833/7b | See Figure 7b in the Reference Standard | 27 | 32 | EN 300 833 V1.2.1 (August 2000) Fixed Radio systems; Point to point Antennas; Antennas for point-to-point fixed radio systems operating in the frequency band 3 GHz to 60 GHz |  |
|  | 833/7c | See Figure 7c in the Reference Standard |  |  |  |  |
| 51.4-52.6 GHz FIXED MOBILE | 833/7b | See Figure 7b in the Reference Standard | 27 | 32 | EN 300 833 V1.2.1 (August 2000) Fixed Radio systems; Point to point Antennas; Antennas for point-to-point fixed radio systems operating in the frequency band 3 GHz to 60 GHz |  |
|  | 833/7c | See Figure 7c in the Reference Standard |  |  |  |  |
| 51.78-57 GHz FIXED MOBILE | 833/7b | See Figure 7b in the Reference Standard | 27 | 32 | EN 300 833 V1.2.1 (August 2000) Fixed Radio systems; Point to point Antennas; Antennas for point-to-point fixed radio systems operating in the frequency band 3 GHz to 60 GHz |  |
|  | 833/7c | See Figure 7c in the Reference Standard |  |  |  |  |
| 57-59 GHz FIXED MOBILE | 833/7b | See Figure 7b in the Reference Standard | 27 | 32 | EN 300 833 V1.2.1 (August 2000) Fixed Radio systems; Point to point Antennas; Antennas for point-to-point fixed radio systems operating in the frequency band 3 GHz to 60 GHz |  |
|  | 833/7c | See Figure 7c in the Reference Standard |  |  |  |  |