HCMFS_DLL description

The HCMFS_DLL has three subroutines:

HCMFS_V2

HCMFS_ANNEX11_1

HCMFS_ANNEX11_2

1st subroutine HCMFS_V2:

Input values:	I_Record1		Tx data according to ANNEX 2B

I_Record2		Rx or PRx data according to ANNEX 2B

I_Record3		PTx data according to ANNEX 2B or empty (P-P)

I_Record4		Rx data according to ANNEX 2B or empty (P-P)

I_DLL_textfile	optional filename (with full path) for an output file

I_Drive		Drive letter of the drive containing the

TOPO, MORPHO and BORDER directories

Output values:		I_O_AREA	overlapping area  (DOUBLE)

I_MD		MD value          (REAL)

I_NFD		NFD value         (REAL)

I_I		interferer level  (REAL)

I_A_TOT	total attenuation (REAL)

I_TD		TD                (REAL)

I_Error		Error value       (LONG)

Additional Error codes:	2000	error in first record (Tx)

2001	error in second record (Rx or PRx)

2002	error in third record (PTx)

2003	error in forth record (Rx)

Order of variables :

SUBROUTINE HCMFS_V2 ( I_O_AREA, I_MD, I_NFD, I_I, I_A_TOT, I_TD, I_Error, &

I_Record1, I_Record2, I_Record3, I_Record4, &

I_DLL_textfile, I_Drive )

2nd subroutine:	HCMFS_ANNEX11_1

Input values:	I_Record1	Data record according to ANNEX 2B

I_Drive	Drive letter of the drive containing the

TOPO, MORPHO and BORDER directories

Output values:	I_Record2	List of countries to coordinate with or

empty, if no coordination is required

I_Error		Error value

Error codes:	0		No error

1		No borderline data found

2		Error in geographical co-ordinates

3		Error in transmitter frequency

4		Error opening a border line data file

5		Error reading border line data

6		Error in antenna height

7		Error in site height

8		Error in input record I_Record1

Order of variables :

SUBROUTINE HCMFS_ANNEX11_1 ( I_Error, I_Record1, I_Record2, I_Drive )

3rd subroutine:	HCMFS_ANNEX11_2

Input values:	I_COO		Co-ordinates of the station (4C, ANNEX 2B)

I_FRE		Frequency of the station (1A ANNEX 2B)

I_C		Country (4B ANNEX 2B)

I_AH		Antenna height (9Y ANNEX 2B)

I_SH		Site height (4Z ANNEX 2B)

I_Drive	Drive letter of the drive containing the

TOPO, MORPHO and BORDER directories

Output values:		I_Record1	List of countries to coordinate with or

empty, if no coordination is required

I_Error		Error value

Error codes:	0	No error

1	No borderline data found

2	Error in geographical co-ordinates

3	Error in transmitter frequency

4	Error opening a border line data file

5	Error reading border line data

6	Error in antenna height

7	Error in site height

Order of variables :

SUBROUTINE HCMFS_ANNEX11_2 ( I_Error, I_COO, I_FRE, I_C, I_AH, I_SH, &

I_Record1, I_Drive )