5. Input values:

- Mode of calculation:	Mandatory, (chapter 3 of User Guide)

- Transmitting frequency:	Mandatory

- Geographical co-ordinates of the transmitter:	Mandatory (including seconds).

- Height of transmitter above sea level:	Optionally. If missing, height is taken

from terrain database.

- Transmitter antenna height:	Mandatory

- Type of antenna horizontal (Tx):	Mandatory; coding see VA01

- Type of antenna vertical (Tx):	Mandatory; coding see VA01

- Azimuth of maximum radiation Tx:	Mandatory. If horiz. Tx antenna is

‘000ND00’, value is not used.

- Elevation angle of main radiation Tx:	Mandatory. . If verti. Tx antenna is

‘000ND00’, value is not used.

- Maximum radiated power:	Mandatory; in dBW.

- Type of Tx antenna:	Mandatory; "E" or "I"

- Channel occupation:	Mandatory. "0" or "1". For some

CMODES not used (default = 0).

- Radius of the service area of the transmitter:	Mandatory. If 0, effective antenna height

of Tx is calculated, if > 0, eff. antenna

height is set to 3 m (for mobiles).

- Designation of emission Tx:	Mandatory for some point to point

calculations, value not used for (border-)

line calculations.

- Distance over sea:	Optionally; in km. If missing, the

distance over sea is calculated if

morphological data is available.

- Sea temperature:	Mandatory; cold or warm ("C" or "W").

C = North sea, W = Mediterranean sea.

- Reception frequency:	Mandatory for point to point

calculations, value not used for (border-)

line calculations.

- Geographical co-ordinates of Rx;	Mandatory for point to point

calculations, value not used for (border-)

line calculations.

- Height of receiver above sea level:	Optionally. If missing, height is taken

from terrain database. Value not used for

(border-) line calculations.

- Receiver antenna height:	Mandatory for some point to point

calculations, value not used for (border-)

line calculations.

- Type of antenna horizontal (Rx):	Mandatory for point to point

calculations, value not used for (border-)

line calculations. Coding see VA01

- Type of antenna vertical (Rx): 	Mandatory for point to point

calculations, value not used for (border-)

line calculations. Coding see VA01

- Azimuth of maximum gain of Rx antenna:	Mandatory for point to point

calculations, value is not used for

(border-) line calculations or if horiz. Rx

antenna is ‘000ND00’.

- Elevation angle of main gain of Rx antenna:	Mandatory for point to point

calculations, value is not used for

(border-) line calculations or if verti. Rx

antenna is ‘000ND00’.

- Gain of Rx-antenna:	Mandatory for point to point

calculations, value is not used for

(border-) line calculations.

- Type of Rx antenna gain:	Mandatory for point to point

calculations ("E" or "I"), value is not

used for (border-) line calculations.

- Designation of emission Rx:	Mandatory for point to point

calculations, value is not used for

(border-) line calculations.

- Depolarisation loss:	Mandatory for point to point

calculations, value is not used for

(border-) line calculations.

- Radius of the service area of the receiver:	Mandatory. If 0, effective antenna height

of Rx is calculated, if > 0, eff. antenna

height is set to 3 m (for mobiles).

- Input-value: permissible field strength:	Optionally, if missing, value is taken

from table in Annex 1 of VA01.

- Input value of correction factor

according frequency difference:	Optionally. Not used for (border-) line

calculations. If missing, value is

calculated.

- Land to calculate from (code of country):	Mandatory

See Preface to the IFL.

- Land to calculate to (code of country): 	Mandatory

See Preface to the IFL.

- Distance to borderline: 	Mandatory for (border-) line

calculations. Not used for point to point

calculations.

negative values = Cross-border range

0 = calculation on borderlines

positive value x= calculation on x km

line.

- Input value of „Maximum cross

border range of harmful interference“:	Optionally for (border-) line

calculations. If missing, value is taken

from table in Annex 1 of VA01.

6. Output values:

- Error value (see list), 0 = no error.

- Distance between point A and B in km (only point to point calculations).

- Height of Tx above sea level (terr. database) in m.

- Height of Rx above sea level (terr. database) (only point to point calculations) in m.

- Information (fields) (see list).

- Eff. antenna height [m] (CCIR) (Tx) (only point to point calculations).

- Eff. antenna height [m] (CCIR) (Rx) (only point to point calculations).

- Total eff. antenna height [m] (CCIR) (only point to point calculations).

- Delta h [m] (CCIR) (only point to point calculations).

- Correction factor k delta h [dB] (CCIR) (only point to point calculations).

- Power [dBW] in direction of receiver (only point to point calculations).

- Free space field strength [dBµV/m] (only point to point calculations).

- Field strength [dBµV/m] (CCIR) land.

- Field strength [dBµV/m] (CCIR) sea.

- Correction according antenna attenuation [dB] (Tx) (only point to point calculations).

- Correction according antenna type [E or I] [dB] (Tx) (only point to point calculations).

- Azimuth (Tx) [degree]

- Elevation (Tx) [degree]

- Direction to Rx [degree].

- Angle vertical Tx to Rx [degree].

- Difference angle vertical (Tx-Rx - elev.) [degree] only point to point calculations).

- Difference angle horizontal (Tx-Rx - azim. Tx) [degree] (only point to point calculations).

- Angle vertical Rx to Tx [degree] (only point to point calculations).

- Difference angle horizontal (Rx-Tx - azim. Rx) [degree] (only point to point calculations).

- Difference angle vertical (Rx-Tx - elev. Rx) [degree] (only point to point calculations).

- Azimuth (Rx) [degree] (only point to point calculations).

- Elevation (Rx) [degree] (only point to point calculations).

- Direction to Tx [degree] (only point to point calculations).

- Clearance angle [degree] (Tx) (only point to point calculations).

- Correction factor according clearance angle [dB] (Tx) (only point to point calculations).

- Clearance angle [degree] (Rx) (only point to point calculations).

- Correction factor according clearance angle [dB] (Rx) (only point to point calculations).

- Distance over sea [km] (calculated or from input value)

- Correction according antenna attenuation (Rx) [dB] (only point to point calculations).

- Correction according antenna type [E or I] (Rx) [dB] (only point to point calculations).

- Delta frequency [kHz] (only point to point calculations).

- Correction factor according delta frequency [dB] (only point to point calculations).

- Channel spacing of Rx [kHz] (only point to point calculations).

- Channel spacing of Tx [kHz] (only point to point calculations).

- Calculated field strength [dBµV/m].

- Permissible field strength [dBµV/m]

- Permissible field strength of table in Annex 1 of VA01 [dBµV/m]

- Max. range of harmful interference [km] (only point to point calculations).

- E.R.P. of reference transmitter [dBW] (only point to point calculations).

- Protection margin (EP - EC) [dB]

- Co-ordinates of (calculated) Tx - point (differs from input value if Tx is a mobile).

- Co-ordinates of (calculated) Rx - point (differs from input value if Rx is a mobile).

HCM_error values:

0	No error

36	Error opening terrain- or morphological data file (data not available)

200	Error in longitude (in 'Point_height' or 'Point_type' subroutine)

210	Error in latitude (in 'Point_height' or 'Point_type' subroutine)

220	Error reading record (in 'Point_height' or 'Point_type' subroutine)

300	Latitude is not in range of 0.0 - 90.0 (in 'Point_height' or

'Point_type' subroutine)

400	Height is missing (-9999) (in 'Point_height' subroutine)

1000	Distance between Tx and Rx = 0. Calculations not possible

1001	Error in geographical coordinates (Tx longitude, degrees)

1002	Error in geographical coordinates (Tx longitude, minutes)

1003	Error in geographical coordinates (Tx longitude, seconds)

1004	Error in geographical coordinates (Tx longitude, E/W)

1005	Error in geographical coordinates (Tx latitude, degrees)

1006	Error in geographical coordinates (Tx latitude, minutes)

1007	Error in geographical coordinates (Tx latitude, seconds)

1008	Error in geographical coordinates (Tx latitude, N/S)

1009	Error in Tx antenna height

1010	Error in transmitting frequency value

1011	Error in transmitting frequency unit

1012	Error in radius of service area of Tx

1013	Error in input value height of Tx site above sea level

1014	Error in geographical coordinates (Rx longitude, degrees)

1015	Error in geographical coordinates (Rx longitude, minutes)

1016	Error in geographical coordinates (Rx longitude, seconds)

1017	Error in geographical coordinates (Rx longitude, E/W)

1018	Error in geographical coordinates (Rx latitude, degrees)

1019	Error in geographical coordinates (Rx latitude, minutes)

1020	Error in geographical coordinates (Rx latitude, seconds)

1021	Error in geographical coordinates (Rx latitude, N/S)

1022	Error in Rx antenna height

1023	Error in reception frequency value

1024	Error in reception frequency unit

1025	C_mode is out of range

1026	Error in input value of permissible field strength

1027	Error in input value of maximum cross border range

1028	The distance is greater than 1000 km. Calculations not possible

1029	Error in radius of Rx service area

1030	Error in input value Rx site height above sea level

1031	Error in Tx elevation

1032	Error in Tx azimuth

1033	Error in type of Tx antenna (E/I)

1034	Error in power

1035	Error in input value of distance over sea

1036	The 'xxx.ALL' borderline file for Tx is missing

1037	The 'xxx.ALL' borderline file for Rx is missing

1038	Error in type of antenna (TR20-08)

1039	Error in input data of correction factor according frequency difference

1040	Channel spacing outside definition range (Rx)

1041	Channel spacing outside definition range (Tx)

1042	Error in Rx elevation

1043	Error in Rx azimuth

1044	Error in Rx type of antenna ("E" or "I")

1045	Error in gain of Rx antenna

1046	Error in input data of depolarization loss

1047	Distance to borderline is too long

1048	Selected line data not available

1049	Error in line data

1050	Non-Agreement frequency and important input data missing

2000	wrong Figure_frequency	(from Get_figure_FS_value)

2001	wrong Time_percentage	(from Get_figure_FS_value)

2002	wrong Sea_temperature		(from Get_figure_FS_value)

2003	wrong Figure_Heff		(from Get_figure_FS_value)

2004	wrong Figure_distance		(from Get_figure_FS_value)

Info(i) values:

1	No height of Tx site is given; height is taken from the terrain database

2	Height of Tx site differs from height of terrain database

3	Height of Tx site differs more than 10%, calculated values may be (extremely)

wrong!

4	Frequency out of range of table in Annex 1

5	Input value of permissible field strength is used

6	Input value of maximum cross border range is used

7	Distance between Tx and Rx is less than both service area radiuses;

field strength is set to 999.9

8	No height of Rx site is given, height is from the terrain database

9	Height of Rx site differs from height of terrain data

10	Rx site height differs more than 10%, calculated values may be (extremely)

wrong!

11	Free space field strength used because distance < 1km

12	Free space field strength is used, because 1st Fresnel zone is free

13	Distance over sea is greater than total distance. Distance between Tx and

Rx is used!

14	Input value of correction factor according frequency difference is used

15	Frequency difference outside definition range; 82 dB is used

16	Calculated distance over sea is set to 0 because of missing morphological

data.

- Channel spacing outside curve range, broadband formula is used.

- Correction factors for the band 380 - 400 MHz are used

7. Using text files for the input:

For simple surrounding programs (e.g. written in MS ACCESS), it is possible to select all input data and write it to a text file. Then the HCMMS_V7 program can be run using the ‘read input from file’ option. In this case, no additional input is requested and the output is written to a text file ‘HCM.TXT’. The format of the input text file differs according to the CMODE. Examples are shown below.

Note: It is possible to queue as many calculation input data as you like. Simply add more lines to the input text file.

The examples can be used as reference for the format of data not listed in Annex 2 of the Agreement 2003.

Example 1 for a data file containing all input for a borderline calculation.

Data	Description / Format

(not part of data!)

| - | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | mode of calculation /N2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 6 | E | 2 | 0 | 0 | 0 | 5 | 1 | N | 0 | 0 | 0 | 0 |  | Tx co-ordinates / C15 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Tx height above sea level / NC4 |
| 0 | 0 | 0 | N | D | 0 | 0 |  |  |  |  |  |  |  |  |  | Tx antenna type horizontal / C7 |
| 0 | 0 | 0 | N | D | 0 | 0 |  |  |  |  |  |  |  |  |  | Tx antenna type vertical / C7 |
|  |  | 0 | . | 0 |  |  |  |  |  |  |  |  |  |  |  | azimuth / D5.1 |
|  |  | 0 | . | 0 |  |  |  |  |  |  |  |  |  |  |  | elevation / D5.1 |
|  |  | 2 | 0 |  |  |  |  |  |  |  |  |  |  |  |  | Tx antenna height / N4 |
|  |  | 1 | 0 | . | 0 |  |  |  |  |  |  |  |  |  |  | radiated power / D6.1 |
| E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Tx type of antenna (E or I) / C1 |
| 0 | 0 | 9 | 5 | 4 | . | 4 | 0 | 0 | 0 | 0 | M |  |  |  |  | Tx frequency / D11.5 + C1 |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | channel occupation / N1 |
| C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | sea temperature (C or W) / C1 |
|  |  | 0 | . | 0 |  |  |  |  |  |  |  |  |  |  |  | distance over sea / DC5.1 |
|  |  |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  | radius of service area / N5 |
|  |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  | distance to borderline (km) / N4 |
| D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | land of Tx (IFL - code) / C3 |
| H | O | L |  |  |  |  |  |  |  |  |  |  |  |  |  | land to calculate to / C3 |
|  | 1 | 9 | . | 0 |  |  |  |  |  |  |  |  |  |  |  | perm. field str. (input) / DC5.1 |
| 1 | 0 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  | max. cbr (input) / C3 |

Format description:

Nx = Number, x = number of digits, right justified.

Cx = Character (letters or numbers), x  = maximum number of characters, left justified.

Dx.y = Decimal numbers, x = total length (including the decimal point), y = number of digits

after the decimal point.

NCx = Same as Nx, but it is possible to leave this field empty (blanks).

DCx.y = Same as Dx.y, but it is possible to leave this field empty (blanks).

Example 2 for a data file containing all input for a normal calculation.

Data	Description / Format

(not part of data!)

|  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | mode of calculation / N2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 6 | E | 2 | 2 | 3 | 3 | 5 | 1 | N | 1 | 5 | 2 | 5 |  | Tx co-ordinates / C15 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Tx height above sea level / NC4 |
| 0 | 0 | 0 | N | D | 0 | 0 |  |  |  |  |  |  |  |  |  | Tx antenna type horizontal / C7 |
| 0 | 0 | 0 | N | D | 0 | 0 |  |  |  |  |  |  |  |  |  | Tx antenna type vertical / C7 |
|  |  | 0 | . | 0 |  |  |  |  |  |  |  |  |  |  |  | azimuth of Tx antenna / D5.1 |
|  |  | 0 | . | 0 |  |  |  |  |  |  |  |  |  |  |  | elevation of Tx antenna / D5.1 |
|  |  | 1 | 0 |  |  |  |  |  |  |  |  |  |  |  |  | Tx antenna height / N4 |
|  |  | 1 | 0 | . | 0 |  |  |  |  |  |  |  |  |  |  | radiated power / D6.1 |
| E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Tx type of antenna (E or I) / C1 |
| 0 | 0 | 1 | 4 | 7 | . | 7 | 7 | 0 | 0 | 0 | M |  |  |  |  | Tx frequency / D11.5 + C1 |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | channel occupation / N1 |
| C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | sea temperature (C or W) / C1 |
|  |  | 0 | . | 0 |  |  |  |  |  |  |  |  |  |  |  | distance over sea / DC5.1 |
|  |  |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  | radius of Tx service area / N5 |
| 0 | 0 | 6 | E | 0 | 0 | 0 | 0 | 5 | 1 | N | 0 | 0 | 0 | 0 |  | Rx coordinates / C15 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rx height above sea level / NC4 |
|  |  | 1 | 0 |  |  |  |  |  |  |  |  |  |  |  |  | Rx antenna height / N4 |
| D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Country code of Tx / C3 |
| H | O | L |  |  |  |  |  |  |  |  |  |  |  |  |  | County code of Rx / C3 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | perm. field str. (input) / DC5.1 |
| 0 | 0 | 1 | 4 | 7 | . | 7 | 7 | 0 | 0 | 0 | M |  |  |  |  | Rx frequency / D11.5 + C1 |
| 1 | 4 | K | 0 |  |  |  |  |  |  |  |  |  |  |  |  | Tx designation of emission / C9 |
| 1 | 4 | K | 0 |  |  |  |  |  |  |  |  |  |  |  |  | Rx designation of emission / C9 |
| 0 | 0 | 0 | N | D | 0 | 0 |  |  |  |  |  |  |  |  |  | Rx antenna type horizontal / C7 |
| 0 | 0 | 0 | N | D | 0 | 0 |  |  |  |  |  |  |  |  |  | Rx antenna type vertical / C7 |
|  |  | 0 | . | 0 |  |  |  |  |  |  |  |  |  |  |  | azimuth of Tx antenna / D5.1 |
|  |  | 0 | . | 0 |  |  |  |  |  |  |  |  |  |  |  | elevation of Rx antenna / D5.1 |
| E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rx type of antenna (E or I) / C1 |
| 0 | 0 | . | 0 |  |  |  |  |  |  |  |  |  |  |  |  | gain of Rx antenna / D4.1 |
| 0 | 0 | . | 0 |  |  |  |  |  |  |  |  |  |  |  |  | depolarization loss / D4.1 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | corr. factor acc. freq. diff. / NC2 |
|  |  |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  | radius of Rx service area / N5 |

8. Output file HCM_X.TXT

Every time the HCMMS_V7.EXE is run, an output text file with a fixed structure is written. This file is the HXM_X.TXT. Specially if you run the EXE with file input, this HCM_X.TXT offers you the possibility to analyze the results with additional software, e.g. with EXCEL.

Contents of HCM_X.TXT

General:

All data items are separated by a semicolon. All input and output data for one calculation is in one long line of text (624 characters). Some data items are blank depending on the input data and the calculation mode.

Data Items in one line:

# Description					Variable name		Input or Output

Calculation mode				C_mode			Input

Tx co-ordinates				Coo_Tx			Input

Site height of Tx station			H_Tx_input			Input

Horiz. antenna type of Tx			Ant_typ_H_Tx		Input

Vert. antenna type of Tx			Ant_typ_V_Tx		Input

Azimuth of Tx antenna			Azi_Tx_input			Input

Elevation of Tx antenna			Ele_Tx_input			Input

Tx antenna height				H_Tx_ant			Input

Maximum radiated power			Max_power			Input

Type of Tx antenna ( E / I )			Type_of_Tx_ant		Input

Tx frequency					TX_frequ			Input

Channel occupation				Chan_occup			Input

Sea temperature				Sea_temperature		Input

Input value of distance over sea		D_sea_input			Input

Radius of Tx service area			Rad_of_Tx_serv_area	Input

Rx co-ordinates				Coo_Rx			Input

Site height of Rx station			H_Rx_input			Input

Rx antenna height				H_Rx_ant			Input

Distance to borderline			D_to_border			Input

Country of Tx					Land_from			Input

Country of Rx					Land_to			Input

Input value of permissible field strength	Perm_FS_input		Input

Rx frequency					RX_frequ			Input

Designation of emission of Tx		Desig_of_Tx_emis		Input

Designation of emission of Rx		Desig_of_Rx_emis		Input

Horiz. antenna type of Rx			Ant_typ_H_Rx		Input

Vert. antenna type of Rx			Ant_typ_V_Rx		Input

Azimuth of Rx antenna			Azi_Rx_input			Input

Elevation of Rx antenna			Ele_Rx_input			Input

Type of Rx antenna ( E / I )			Type_of_Rx_ant		Input

Rx antenna gain				Rx_ant_gain			Input

Depolarization loss				Depol_loss			Input

Correction factor acc. frequ. difference	Cor_fact_frequ_diff		Input

# Description					Variable name		Input or Output

Radius of Rx service area			Rad_of_Rx_serv_area	Input

Input of maximum cross-border-range	Max_CBR_D-input		Input

Version number				Version			Output

Error value					HCM_error			Output

Calculated Tx co-ordinates			Coo_Tx_new			Output

Calculated field strength			Calculated_FS		Output

Distance Tx position – Rx position		Distance			Output

Direction Tx position  Rx position	Dir_Tx_Rx			Output

Permissible field strength			Perm_FS			Output

Protection margin				Prot_margin			Output

Calculated distance over sea			D_sea_calculated		Output

Calculated Rx co-ordinates			Coo_Rx_new			Output

ERP of reference transmitter			ERP_ref_Tx			Output

Cross-border-range				CBR_D			Output

20 info values, ‘T’ RUE or ‘F’ ALSE	Info				Output

Site height of Tx from terrain database	H_Datab_Tx			Output

Site height of Rx from terrain database	H_Datab_Rx			Output

Transmitter terrain clearance angle		Tx_TCA			Output

Correction acc. Tx terrain clearance angle	Tx_TCA_corr			Output

Receiver terrain clearance angle		Rx_TCA			Output

Correction acc. Rx terrain clearance angle	Rx_TCA_corr			Output

Effective antenna height of Tx		Heff_Tx			Output

Effective antenna height of Rx		Heff_Rx			Output

Total effective antenna height		Heff				Output

Terrain irregularity				Dh				Output

Correction according terrain irregularity	Dh_corr			Output

Land field strength				Land_FS			Output

Sea field strength				Sea_FS			Output

Correction acc. Tx antenna type (E / I )	Tx_ant_type_corr		Output

Correction acc. Tx antenna type (H and V)	Tx_ant_corr			Output

Direction Rx position  Tx position	Dir_Rx_Tx			Output

Vertical angle Tx  Rx			V_angle_Tx_Rx		Output

Horizontal difference angle Tx  Rx	H_diff_angle_Tx_Rx		Output

Vertical difference angle Tx  Rx		V_diff_angle_Tx_Rx		Output

Vertical angle Rx  Tx			V_angle_Rx_Tx		Output

Horizontal difference angle Rx  Tx	H_diff_angle_Rx_Tx		Output

Vertical difference angle Rx  Tx		V_diff_angle_Rx_Tx		Output

Correction acc. Rx antenna type (E / I )	Rx_ant_type_corr		Output

Correction acc. Rx antenna type (H and V)	Rx_ant_corr			Output

Frequency difference				Delta_frequency		Output

Permissible field strength from table		Perm_FS_from_table		Output

Correction factor acc. frequency difference	Corr_delta_f			Output

Channel spacing of Rx			Channel_sp_Rx		Output

Channel spacing of Tx			Channel_sp_Tx		Output

Radiated power in direction of Rx		Power_to_Rx			Output

Free space field strength			Free_Space_FS		Output