Welcone to

HCM_MS.exe

the simple surrounding software for most versions of HCM DLLs.

# General:

This is “portable” software, meaning nothing has to be installed. Just copy the .exe to a directory of your choice and supplement it with some versions of HCM DLLs. At the first run you will have to initialise the information where TOPO/MORPHO/BORDER data can be found.

This software can be used in interactive or batch mode. When started from the commandline it will run in batch-mode and exactly one argument has to be supplied: the (path and) name of the input file. The results will be written to a subdirectory “HCM” to where the software is located.

The output will consist of two different files: HCM.txt and HCM_X.txt. The first has the same format as allready known from the old Version (aka debug.txt), the HCM_X as described in the last chapter. This HCM_X (format) will also be accepted as input (for “re-calculation”).

# Using text files for input:

There are two ways to use files for input:

- give the file as argument on command line

- select the file from mainmenu using button “read file with input data”

both methods accept files with mixed data for

- point2point (C_MODE >= 0)	(see example 1)

- point2line (C_MODE <0)	(see example 2)

A file containing data for several calculations can be created by concatenating the needed data for each calculation without separator:

begin of file

first field calc #1

...

last field calc #1

first field calc #2

...

...

last field calc #n

end of file

as alternative “csv” can be used where all fields for one calculation are in one line, separated by a semicolon (;). No need to pad values with spaces, even ;; is allowed.

begin of file

field_1-calc_1;...;field_n-calc_1

....

field_1-calcm;...;field_n-calc_m

end of file

Example 1 for a data file containing all input for a borderline calculation.

| Data |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Description /Format (not part of data!) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | - | 9 |  |  |  |  |  |  |  |  |  |  |  |  |  | mode of calculation /N3 |
| 0 | 0 | 6 | E | 2 | 0 | 0 | 0 | 5 | 1 | N | 0 | 0 | 0 | 0 |  | Tx co-ordinates /C15 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Tx height above sea level /NC4 |
| 0 | 0 | 0 | N | D | 0 | 0 |  |  |  |  |  |  |  |  |  | Tx antenna type horizontal /C7 |
| 0 | 0 | 0 | N | D | 0 | 0 |  |  |  |  |  |  |  |  |  | Tx antenna type vertical /C7 |
|  |  | 0 | . | 0 |  |  |  |  |  |  |  |  |  |  |  | azimuth /D5.1 |
|  |  | 0 | . | 0 |  |  |  |  |  |  |  |  |  |  |  | elevation /D5.1 |
|  |  | 2 | 0 |  |  |  |  |  |  |  |  |  |  |  |  | Tx antenna height /N4 |
|  |  | 1 | 0 | . | 0 |  |  |  |  |  |  |  |  |  |  | radiated power /D6.1 |
| E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Tx type of antenna (E or I) /C1 |
| 0 | 0 | 9 | 5 | 4 | . | 4 | 0 | 0 | 0 | 0 | M |  |  |  |  | Tx frequency /D11.5 + C1 |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | channel occupation / N1 |
| C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | sea temperature (C or W) /C1 |
|  |  | 0 | . | 0 |  |  |  |  |  |  |  |  |  |  |  | distance over sea /DC5.1 |
|  |  |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  | radius of service area /N5 |
|  |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  | distance to borderline (km) /N4 |
| D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | land of Tx (IFL - code) /C3 |
| H | O | L |  |  |  |  |  |  |  |  |  |  |  |  |  | land to calculate to /C3 |
|  | 1 | 9 | . | 0 |  |  |  |  |  |  |  |  |  |  |  | perm. field str. (input) /DC5.1 |
| 1 | 0 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  | max. cbr (input) /C3 |
| 1 | 4 | K | 0 | F | 3 | E |  |  |  |  |  |  |  |  |  | Tx designation of emission /C9 |

Example 2 for a data file containing all input for a normal calculation.

| Data |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Description /Format (not part of data!) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  | mode of calculation /N3 |
| 0 | 0 | 6 | E | 0 | 0 | 0 | 0 | 5 | 0 | N | 0 | 0 | 0 | 0 |  | Tx co-ordinates /C15 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Tx height above sea level /NC4 |
| 0 | 0 | 0 | N | D | 0 | 0 |  |  |  |  |  |  |  |  |  | Tx antenna type horizontal /C7 |
| 0 | 0 | 0 | N | D | 0 | 0 |  |  |  |  |  |  |  |  |  | Tx antenna type vertical /C7 |
|  |  | 0 | . | 0 |  |  |  |  |  |  |  |  |  |  |  | azimuth of Tx antenna /D5.1 |
|  |  | 0 | . | 0 |  |  |  |  |  |  |  |  |  |  |  | elevation of Tx antenna /D5.1 |
|  |  | 1 | 0 |  |  |  |  |  |  |  |  |  |  |  |  | Tx antenna height /N4 |
|  |  | 1 | 0 | . | 0 |  |  |  |  |  |  |  |  |  |  | radiated power /D6.1 |
| E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Tx type of antenna (E or I) /C1 |
| 0 | 0 | 1 | 4 | 7 | . | 7 | 7 | 0 | 0 | 0 | M |  |  |  |  | Tx frequency /N11+ C1 |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | channel occupation /N1 |
| C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | sea temperature (C or W) /C1 |
|  |  | 0 | . | 0 |  |  |  |  |  |  |  |  |  |  |  | distance over sea /DC5.1 |
|  |  |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  | radius of Tx service area /N5 |
| 0 | 0 | 6 | E | 2 | 0 | 0 | 0 | 5 | 0 | N | 0 | 0 | 0 | 0 |  | Rx coordinates /C15 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rx height above sea level /NC4 |
|  |  | 1 | 0 |  |  |  |  |  |  |  |  |  |  |  |  | Rx antenna height /N4 |
| L | U | X |  |  |  |  |  |  |  |  |  |  |  |  |  | country from /C3 |
| D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | country to /C3 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | perm. Field str. (input) /DC5.1 |
| 0 | 0 | 1 | 4 | 7 | . | 7 | 7 | 0 | 0 | 0 | M |  |  |  |  | Rx frequency /N11+ C1 |
| 1 | 4 | K | 0 | F | 3 | E |  |  |  |  |  |  |  |  |  | Tx designation of emission /C9 |
| 1 | 4 | K | 0 | F | 3 | E |  |  |  |  |  |  |  |  |  | Rx designation of emission /C9 |
| 0 | 0 | 0 | N | D | 0 | 0 |  |  |  |  |  |  |  |  |  | Rx antenna type horizontal /C7 |
| 0 | 0 | 0 | N | D | 0 | 0 |  |  |  |  |  |  |  |  |  | Rx antenna type vertical /C7 |
|  |  | 0 | . | 0 |  |  |  |  |  |  |  |  |  |  |  | azimuth of Tx antenna /D5.1 |
|  |  | 0 | . | 0 |  |  |  |  |  |  |  |  |  |  |  | elevation of Rx antenna /D5.1 |
| E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rx type of antenna (E or I) /C1 |
| 0 | 0 | . | 0 |  |  |  |  |  |  |  |  |  |  |  |  | gain of Rx antenna /D4.1 |
| 0 | 0 | . | 0 |  |  |  |  |  |  |  |  |  |  |  |  | depolarization loss /D4.1 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | corr. factor acc. freq. diff. /NC4 |
|  |  |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  | radius of Rx service area /N5 |

Format description:

Nx = Number, x = number of digits, right justified.

Cx = Character (letters or numbers), x  = maximum number of characters, left justified.

Dx.y = Decimal numbers, x = total length (including the decimal point), y = number of digits

after the decimal point.

NCx = Same as Nx, but it is possible to leave this field empty (blanks).

DCx.y = Same as Dx.y, but it is possible to leave this field empty (blanks).

padding of fields is mandatory

# Format of HCM_X.TXT

General:

All input and output data values for one calculation in one long line of text (csv format, fields separated by “;”, max 624 characters). Some data items are blank depending on the input data and the calculation mode.

This file (format) will also be accepted as input (for “re-calculation”).

Data Items in one line:

# Description					Variable name		Input or Output

Calculation mode				C_mode			Input

Tx co-ordinates				Coo_Tx			Input

Site height of Tx station			H_Tx_input			Input

Horiz. antenna type of Tx			Ant_typ_H_Tx		Input

Vert. antenna type of Tx			Ant_typ_V_Tx		Input

Azimuth of Tx antenna			Azi_Tx_input			Input

Elevation of Tx antenna			Ele_Tx_input			Input

Tx antenna height				H_Tx_ant			Input

Maximum radiated power			Max_power			Input

Type of Tx antenna ( E / I )			Type_of_Tx_ant		Input

Tx frequency					TX_frequ			Input

Channel occupation				Chan_occup			Input

Sea temperature				Sea_temperature		Input

Input value of distance over sea		D_sea_input			Input

Radius of Tx service area			Rad_of_Tx_serv_area	Input

Rx co-ordinates				Coo_Rx			Input

Site height of Rx station			H_Rx_input			Input

Rx antenna height				H_Rx_ant			Input

Distance to borderline			D_to_border			Input

Country of Tx					Land_from			Input

Country of Rx					Land_to			Input

Input value of permissible field strength	Perm_FS_input		Input

Rx frequency					RX_frequ			Input

Designation of emission of Tx		Desig_of_Tx_emis		Input

Designation of emission of Rx		Desig_of_Rx_emis		Input

Horiz. antenna type of Rx			Ant_typ_H_Rx		Input

Vert. antenna type of Rx			Ant_typ_V_Rx		Input

Azimuth of Rx antenna			Azi_Rx_input			Input

Elevation of Rx antenna			Ele_Rx_input			Input

Type of Rx antenna ( E / I )			Type_of_Rx_ant		Input

Rx antenna gain				Rx_ant_gain			Input

Depolarization loss				Depol_loss			Input

Correction factor acc. frequ. difference	Cor_fact_frequ_diff		Input

# Description					Variable name		Input or Output

Radius of Rx service area			Rad_of_Rx_serv_area	Input

Input of maximum cross-border-range	Max_CBR_D-input		Input

Version number				Version			Output

Error value					HCM_error			Output

Calculated Tx co-ordinates			Coo_Tx_new			Output

Calculated field strength			Calculated_FS			Output

Distance Tx position – Rx position		Distance			Output

Direction Tx position  Rx position	Dir_Tx_Rx			Output

Permissible field strength			Perm_FS			Output

Protection margin				Prot_margin			Output

Calculated distance over sea			D_sea_calculated		Output

Calculated Rx co-ordinates			Coo_Rx_new			Output

ERP of reference transmitter			ERP_ref_Tx			Output

Cross-border-range				CBR_D			Output

20 info values, ‘T’ RUE or ‘F’ ALSE	Info				Output

Site height of Tx from terrain database	H_Datab_Tx			Output

Site height of Rx from terrain database	H_Datab_Rx			Output

Transmitter terrain clearance angle		Tx_TCA			Output

Correction acc. Tx terrain clearance angle	Tx_TCA_corr			Output

Receiver terrain clearance angle		Rx_TCA			Output

Correction acc. Rx terrain clearance angle	Rx_TCA_corr			Output

Effective antenna height of Tx		Heff_Tx			Output

Effective antenna height of Rx		Heff_Rx			Output

Total effective antenna height		Heff				Output

Terrain irregularity				Dh				Output

Correction according terrain irregularity	Dh_corr			Output

Land field strength				Land_FS			Output

Sea field strength				Sea_FS			Output

Correction acc. Tx antenna type (E / I )	Tx_ant_type_corr		Output

Correction acc. Tx antenna type (H and V)	Tx_ant_corr			Output

Direction Rx position  Tx position	Dir_Rx_Tx			Output

Vertical angle Tx  Rx			V_angle_Tx_Rx		Output

Horizontal difference angle Tx  Rx	H_diff_angle_Tx_Rx		Output

Vertical difference angle Tx  Rx		V_diff_angle_Tx_Rx		Output

Vertical angle Rx  Tx			V_angle_Rx_Tx		Output

Horizontal difference angle Rx  Tx	H_diff_angle_Rx_Tx		Output

Vertical difference angle Rx  Tx		V_diff_angle_Rx_Tx		Output

Correction acc. Rx antenna type (E / I )	Rx_ant_type_corr		Output

Correction acc. Rx antenna type (H and V)	Rx_ant_corr			Output

Frequency difference				Delta_frequency		Output

Permissible field strength from table		Perm_FS_from_table		Output

Correction factor acc. frequency difference	Corr_delta_f			Output

Channel spacing of Rx			Channel_sp_Rx		Output

Channel spacing of Tx			Channel_sp_Tx		Output

Radiated power in direction of Rx		Power_to_Rx			Output

Free space field strength			Free_Space_FS		Output

# n.b.:

This new generation of HCM_MS.exe is internally using the HCM-DLL to give exactly the same results for .exe as for .dll (history shows there have been differences due to compiler settings) and minimize the maintenance overhead.

general:

compatible with Windows-XP/Vista/7/8.x/10 32/64 bit

only applicable for HCM Version 7

additional features:

°) DLL-Version selectable

°) "settings" mask

°) supports new C#DLLs

requirements:

min. .net runtime 2.0

TOPO/MORPHO*/BORDER data

dforrt.dll for Versions <7.20

HCM DLLs as wanted

please put HCM DLLs (best named like on server) in same directory where HCM_MS.exe resides.

if only one DLL is found, no dialog will be shown. HCMMS_V7_DLL.dll mandatory for batch mode

To do:

no popup-masks on error in batch mode.

remarks:

DLL version selection only possible on startup (vb.net problem)

only one running instance, except batch mode (vb.net problem)

* if no MORPHO found, no automatic sea-path detection possible --> use input value